# Inner Current Loop Equation Derivation

Two-Stage Interleaved CCM PFC Design — Detailed Theory, Laplace-Domain Derivation and Time-Domain Basis

This document contains the complete explanation previously provided, formatted as a Word document for engineering review and reuse.

I reviewed your attached Word document. It defines the inner current-loop plant as the full duty-to-inductor-current transfer function Gid(s)=iL_hat/d_hat, then builds the current loop as:

Ti(s)=Gid,full(s)·RCS·Gmi(s)·Fm·HCS(s)

with:

Fm=1/VRAMP

and HCS(s) as the current-sense filter, Gmi(s)=gmiZiea(s) as the Type-II OTA current compensator, and RCS as the current-sense resistor. This is exactly the structure shown on page 1 of the document.

One important clarification: a loop equation for a boost PFC cannot be derived with literally zero modeling assumptions, because any transfer function such as Gid(s) already requires a model. What I can do is derive it without dropping terms inside the selected model: CCM averaged boost converter, inductor DCR rL, output capacitor ESR rC, finite load RLOAD, and small-signal linearization about one operating point. No high-frequency pole, ESR zero, load damping, or duty-dependent term is removed.

# 1. What inner current loop controls

For each PFC phase, the inner loop controls the boost inductor current.

The power stage has this signal path:

d_hat(s) → iL_hat(s)

The plant is:

Gid(s)=iL_hat(s)/d_hat(s)

Then the sensed current becomes:

vCS_hat(s)=RCS·HCS(s)·iL_hat(s)

The current error amplifier creates the PWM control command:

vc_hat(s)=Gmi(s)·verr_hat(s)

The PWM modulator converts command voltage into duty perturbation:

d_hat(s)=Fm vc_hat(s)

where:

Fm=1/VRAMP

Therefore the open current-loop gain is:

Ti(s)=Gid(s)·RCS·HCS(s)·Gmi(s)·1/VRAMP

Reordered:

Ti(s)=Gid(s)·RCS·Gmi(s)·Fm·HCS(s)

That matches the structure in your attachment.

# 2. Boost power-stage averaged model

Use one interleaved phase. The two phases affect the voltage-loop and input-current ripple cancellation, but the inner current loop plant per phase is derived from one boost inductor.

Define:

D = steady-state duty cycle

D' = 1-D

iL = inductor current

vo = output voltage

vC = ideal capacitor voltage behind ESR

rL = inductor DCR

rC = output capacitor ESR

R = RLOAD

C = COUT

During switch ON, the inductor charges from input and the output is supplied by capacitor/load. During switch OFF, the inductor transfers energy to the output.

Using duty-cycle averaging:

L·diL/dt=vin-rL·iL-D'·vo

This is the averaged inductor equation.

The capacitor/load equation must include ESR correctly. The averaged diode/output current delivered to the output node is:

id=D'·iL

The capacitor/load relation is:

C(R+rC)·dvC/dt=R·D'·iL-vC

The output voltage is not simply vC when ESR is included. It is:

vo=[R/(R+rC)]·vC + [RrC/(R+rC)]·D'·iL

These three equations are the exact averaged large-signal model for the selected CCM model with rL, rC, R, L, and C retained.

# 3. Apply small-signal perturbation

Perturb every variable around its steady-state operating point:

iL=IL+iL_hat

vC=VC+vC_hat

vo=VO+vo_hat

d=D+d_hat

D'=1-D-d_hat

So:

D'iL=(1-D-d_hat)(IL+iL_hat)

Expanding:

D'iL=(1-D)IL+(1-D)iL_hat-IL d_hat-d_hat iL_hat

The product d_hat iL_hat is second-order small-signal. In linear small-signal theory, it is removed because the transfer function is the first-order derivative around the operating point.

So:

widehat(D'iL)=D' iL_hat-IL d_hat

This is where the small-signal model comes from.

# 4. Small-signal capacitor equation

Start from:

C(R+rC)·dvC/dt=R·D'·iL-vC

Small-signal form:

C(R+rC)·dvC_hat/dt=R(D' iL_hat-IL d_hat)-vC_hat

Move to Laplace domain:

C(R+rC)s vC_hat=R D' iL_hat-R IL d_hat-vC_hat

Therefore:

[C(R+rC)s+1]vC_hat=R D' iL_hat-R IL d_hat

So:

vC_hat(s)=[R D' iL_hat(s)-R IL d_hat(s)]/[1+sC(R+rC)]

# 5. Small-signal output voltage equation

Start from:

vo=[R/(R+rC)]vC+[RrC/(R+rC)]D'iL

Small-signal form:

vo_hat=[R/(R+rC)]vC_hat+[RrC/(R+rC)](D' iL_hat-IL d_hat)

Substitute vC_hat:

vo_hat=[R/(R+rC)]·[R D' iL_hat-R IL d_hat]/[1+sC(R+rC)] + [RrC/(R+rC)](D' iL_hat-IL d_hat)

Factor:

D' iL_hat-IL d_hat

vo_hat=[R/(R+rC)]·[R/(1+sC(R+rC))+rC]·(D' iL_hat-IL d_hat)

Put the bracket over a common denominator:

R/[1+sC(R+rC)] + rC = [R+rC+sCrC(R+rC)]/[1+sC(R+rC)]

Since:

R+rC+sCrC(R+rC)=(R+rC)(1+sCrC)

Then:

vo_hat=[R/(R+rC)]·[(R+rC)(1+sCrC)]/[1+sC(R+rC)]·(D' iL_hat-IL d_hat)

Cancel R+rC:

vo_hat=R·[1+sCrC]/[1+sC(R+rC)]·(D' iL_hat-IL d_hat)

This shows the ESR zero term 1+sCrC, but the final Gid form rearranges it into the numerator zero shown in your document.

# 6. Small-signal inductor equation

Start from:

L·diL/dt=vin-rL iL-D'vo

For control-to-current transfer function Gid(s), input-voltage perturbation is set to zero:

vin_hat=0

Small-signal form:

L·diL_hat/dt = -rL iL_hat -D' vo_hat + Vo d_hat

Laplace transform:

Ls iL_hat=-rL iL_hat-D' vo_hat+Vo d_hat

Move current term to the left:

(Ls+rL)iL_hat+D'vo_hat=Vo d_hat

Substitute vo_hat:

(Ls+rL)iL_hat + D'R·[1+sCrC]/[1+sC(R+rC)]·(D' iL_hat-IL d_hat) = Vo d_hat

Multiply both sides by:

1+sC(R+rC)

[(Ls+rL)(1+sC(R+rC)) + D'^2R(1+sCrC)] iL_hat = [Vo(1+sC(R+rC)) + D'RIL(1+sCrC)] d_hat

Therefore:

Gid(s)=iL_hat/d_hat = [Vo(1+sC(R+rC)) + D'RIL(1+sCrC)] / [(Ls+rL)(1+sC(R+rC)) + D'^2R(1+sCrC)]

This is an exact small-signal expression before steady-state substitutions.

# 7. Use steady-state boost relations to express numerator with VOUT

At the operating point:

Vo=Vin/D'

and the output current is:

Io=Vo/R

The average diode/output current is:

D'IL=Io=Vo/R

So:

IL=Vo/(RD')

Therefore:

D'RIL=Vo

Substitute into the numerator:

Vo(1+sC(R+rC)) + Vo(1+sCrC)

=Vo[2+sC(R+rC)+sCrC]

=Vo[2+sC(R+2rC)]

Factor:

2+sC(R+2rC) = C(R+2rC)[s+2/(C(R+2rC))]

Since:

2/[C(R+2rC)] = 1/[C(R/2+rC)]

So numerator zero is:

ωz=1/[C(R/2+rC)]

This is exactly the zero shown in your document:

s+1/[C(RLOAD/2+rC)]

Important: this is not the simple capacitor ESR zero:

ωESR=1/(CrC)

It includes RLOAD/2, which is why the document correctly says it is not the ESR zero.

# 8. Denominator expansion

The denominator is:

(Ls+rL)(1+sC(R+rC)) + D'^2R(1+sCrC)

Expand first term:

(Ls+rL)(1+sC(R+rC))

=Ls+rL+s^2LC(R+rC)+s rL C(R+rC)

Second term:

D'^2R(1+sCrC)=D'^2R+sD'^2RCrC

Add them:

Den(s)=s^2LC(R+rC) + s[L+rLC(R+rC)+D'^2RCrC] + [rL+D'^2R]

Divide the denominator by:

LC(R+rC)

Then:

Den(s)=s^2+a1s+a0

where:

a1={C[rL(R+rC)+RrC(1-D)^2]+L}/{LC(R+rC)}

and:

a0={ (1-D)^2R+rL }/{LC(R+rC)}

This matches the a1 and a0 structure shown on page 1 of your document.

# 9. Final full duty-to-current plant

After numerator and denominator normalization, the full control-to-inductor-current plant is:

Gid(s)=iL_hat/d_hat = [VOUT/L] · [(R+2rC)/(R+rC)] · [s+ωz]/[s^2+a1s+a0]

where:

ωz=1/[C(R/2+rC)]

a1={C[rL(R+rC)+RrC(1-D)^2]+L}/{LC(R+rC)}

and:

a0={ (1-D)^2R+rL }/{LC(R+rC)}

Using your document notation:

R=RLOAD

C=COUT

rC=capacitor ESR

rL=inductor DCR

D=1-Vin,pk/VOUT

So the final plant is:

Gid(s)= [VOUT/L] · [(RLOAD+2rC)/(RLOAD+rC)] · [s+1/(COUT(RLOAD/2+rC))]/[s^2+a1s+a0]

This is the full form in your attachment.

# 10. Convert the power-stage plant into current-loop gain

The current-loop power-stage block is not just Gid(s). The complete loop includes current sensing, compensation, and PWM gain.

Current sense:

vCS_hat(s)=RCS iL_hat(s)

Current-sense filter:

HCS(s)=1/(1+s/ωRC)

where:

ωRC=1/(RfCf)

Your document uses:

Rf=2kΩ

Cf=470pF

So:

fRC=1/(2π RfCf)

fRC=1/(2π·2000·470×10^-12)

fRC≈169kHz

Current compensator:

Gmi(s)=gmiZiea(s)

PWM modulator:

Fm=1/VRAMP

Therefore:

Ti(s)=Gid(s)·RCS·Gmi(s)·1/VRAMP·HCS(s)

Substitute the full Gid(s):

Ti(s)= [VOUT/L] · [(RLOAD+2rC)/(RLOAD+rC)] · [s+ωz]/[s^2+a1s+a0] · RCS · 1/VRAMP · Gmi(s) · 1/(1+s/ωRC)

or:

Ti(s)= [VOUT/L] · [(RLOAD+2rC)/(RLOAD+rC)] · [s+ωz]/[s^2+a1s+a0] · [RCS/VRAMP] · Gmi(s) · 1/(1+s/ωRC)

This is the expanded inner current-loop equation.

# 11. Meaning of each term

## Term 1 — Inductor current gain

VOUT/L

Duty perturbation changes how much output voltage is applied across the inductor. A larger VOUT gives stronger current response. A larger L slows the current response.

## Term 2 — ESR/load scaling

(RLOAD+2rC)/(RLOAD+rC)

This appears because the duty perturbation affects both inductor current and output capacitor/load current, while output capacitor ESR feeds part of that current immediately into output-voltage perturbation.

If rC=0, this term becomes:

R/R=1

So it disappears only when ESR is zero.

## Term 3 — Numerator zero

s+ωz

where:

ωz=1/[C(R/2+rC)]

This zero comes from output capacitor/load dynamics feeding back into the inductor equation through vo_hat.

It is not:

1/(CrC)

Therefore it is not the conventional capacitor ESR zero.

## Term 4 — Denominator

s^2+a1s+a0

This is the second-order boost power-stage current plant.

The a1 term is damping:

a1={C[rL(R+rC)+RrC(1-D)^2]+L}/{LC(R+rC)}

It includes:

rL

rC

RLOAD

D

This is why your document notes that rL critically changes Q.

The a0 term is the natural-frequency-squared term:

a0={ (1-D)^2RLOAD+rL }/{LC(RLOAD+rC)}

If rL is ignored, damping and natural frequency can be mispredicted.

# 12. Why D appears in the current-loop equation

Boost converter duty affects output coupling through:

D'=1-D

When duty increases, the OFF time decreases. The inductor spends less time delivering energy to the output.

The output-side coupling is proportional to:

D'iL

Small-signal perturbation gives:

widehat(D'iL)=D'iL_hat-ILd_hat

This term creates duty dependence in both:

a1

and:

a0

Specifically:

(1-D)^2

appears because current perturbation must pass through the output diode interval twice: once through output-current injection and once through output-voltage feedback into the inductor equation.

# 13. Why the inner current loop uses RCS/VRAMP

The control IC does not directly compare amperes to duty. It compares voltages.

Current sensing converts inductor current to voltage:

vCS=iL RCS

PWM ramp converts control voltage to duty:

d=vc/VRAMP

So the loop gain must include:

RCS/VRAMP

This is why your document writes the current loop with:

RCS/VRAMP

inside the loop gain.

For your values:

RCS=15mΩ

VRAMP=5V

RCS/VRAMP=0.015/5

RCS/VRAMP=0.003

This is a large attenuation, so the OTA compensator must provide gain.

# 14. Inner current-loop Type-II compensator

Your document says the current loop uses a Type-II OTA compensator with:

fz=1kHz

fp=26kHz

and current-loop crossover target:

fc=8kHz

A common OTA Type-II form is:

Gmi(s)=gmiZiea(s)

with compensator impedance shaped as:

Ziea(s)=RC · [(1+s/ωz)/(s/ωz)] · [1/(1+s/ωp)]

Equivalently:

Gmi(s)=gmiRC · [(1+s/ωz)/(s/ωz)] · [1/(1+s/ωp)]

This gives:

one integrator for high DC gain,

one zero for phase boost,

one high-frequency pole for noise attenuation.

So final current-loop gain becomes:

Ti(s)=Gid(s) · [RCS/VRAMP] · gmiRC · [(1+s/ωz)/(s/ωz)] · [1/(1+s/ωp)] · [1/(1+s/ωRC)]

where:

ωz=2π(1kHz)

ωp=2π(26kHz)

ωRC=2π(169kHz)

# 15. Final derived inner current-loop equation

Putting everything together:

Ti(s)= [VOUT/L] · [(RLOAD+2rC)/(RLOAD+rC)] · [s+1/(COUT(RLOAD/2+rC))]/[s^2+a1s+a0] · [RCS/VRAMP] · Gmi(s) · [1/(1+s/ωRC)]

with:

a1={COUT[rL(RLOAD+rC)+RLOADrC(1-D)^2]+L}/{L COUT(RLOAD+rC)}

a0={ (1-D)^2RLOAD+rL }/{L COUT(RLOAD+rC)}

ωz=1/[COUT(RLOAD/2+rC)]

HCS(s)=1/(1+s/ωRC)

Gmi(s)=gmiZiea(s)

This is the complete inner current-loop derivation consistent with your attached document. The document later uses the same inner-loop structure to calculate uncompensated current-loop gain at 8 kHz for all operating points and then design the Type-II compensator values.

Source reference: user-provided Word document “Voltage Loop Equation 2(4).docx”; inner current-loop equation structure appears on page 1.