| Design outcome | Result | Where |
| --- | --- | --- |
| Selected boost inductor core | Edge 0059071A2, 3-stack, 47 turns | Step 13 |
| No-load inductance | L0 = 371.1 / 404.2 / 437.4 µH (min / nom / max) | Step 13.3 |
| Full-load flux range at 90 Vac | 0.3007 T to 0.4337 T | Step 13.5 |
| Low-line loss estimate | Ptotal = 3.605 W at 25°C; 4.090 W at 100°C | Step 13.7 |
| Time-domain core-loss model | Pcore(B) = k·B^n, n = 2.3000, k = 982.81 | Step 14.3 |
| Selected output capacitor bank | 5 × 470 µF, 450 V Panasonic EEUFC series | Step 15.7 |
| Output capacitor verification | Ripple current PASS; worst-case Tcap = 53.3°C | Step 15.9 |

Nomenclature

| Symbol | Description | Value / Units |
| --- | --- | --- |
| Lϕ,target | Per-phase boost inductance target | 240 µH |
| Iϕ,avg@crest | Per-phase crest-average current | A |
| Iϕ,rms | Per-phase inductor RMS current | A |
| AL | Inductance factor | nH/T² |
| Ae / Wa | Effective magnetic area / window area | mm² |
| N | Number of turns | turns |
| ΔIL,pp | Inductor peak-to-peak ripple current | A |
| Bac,pk | Peak AC flux density | T |
| Bdc | DC flux density | T |
| FFcu | Copper fill factor | % |
| Pcu / Pcore / Ptotal | Copper loss / core loss / total loss | W |
| Choldup / Cripple | Capacitance required for hold-up / ripple | µF |
| ESRpar | Parallel ESR of capacitor bank | mΩ |

Step 13 — Boost Inductor Magnetic Design

Step 13.1 — Reference Operating Point and Design Targets (Low Line, Full Load)

| Vout | 394 Vdc |
| --- | --- |
| fsw | 70 kHz |
| Vin,rms (low line) | 90 Vac @ 60 Hz |
| Vin,pk = √2 · Vin,rms | 127.2792 V |
| Dpk = 1 − (Vin,pk/Vout) | 0.6767 |
| Lϕ,target (low line, full load) | 240 µH |
| Iϕ,avg@crest = Ipk,line/2 | 14.1522 A |
| Iϕ,rms (used for copper loss) | 10.0700 A |
| ∆IL,pp@crest = Vin,pk·Dpk/(Lϕ·fsw) | 5.1269 A |

Step 13.0 — Top Core Candidates: Sizing Engine Results

The engine evaluated all catalog cores for the selected material family and ranked the top 15 candidates. The highlighted row is the approved design carried forward to Steps 13.1–14.

| # | Part number | µ | Stacks | N | FFcu% | ∆T °C | Ptotal W | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0059071A2 | 60 | 3 | 47 | 22 | 32.6 | 3.64 | PASS |
| 2 | 0059555A2 | 40 | 3 | 53 | 25 | 33.8 | 3.77 | PASS |
| 3 | 0059550A2 | 26 | 3 | 60 | 28 | 34.9 | 3.90 | PASS |
| 4 | 0059553A2 | 75 | 3 | 44 | 21 | 35.4 | 3.92 | PASS |
| 5 | 0059076A2 | 60 | 3 | 47 | 18 | 32.7 | 3.94 | PASS |
| 6 | 0059391A2 | 40 | 2 | 42 | 25 | 26.9 | 3.78 | PASS |
| 7 | 0059392A2 | 60 | 2 | 35 | 21 | 27.4 | 3.82 | PASS |
| 8 | 0059390A2 | 26 | 2 | 50 | 29 | 27.2 | 3.85 | PASS |
| 9 | 0059083A2 | 60 | 2 | 46 | 22 | 33.7 | 4.26 | PASS |
| 10 | 0059451A2 | 60 | 2 | 37 | 18 | 27.9 | 4.08 | PASS |
| 11 | 0059729A2 | 75 | 1 | 38 | 35 | 36.0 | 5.05 | PASS |
| 12 | 0059193A2 | 75 | 1 | 43 | 35 | 38.5 | 5.64 | PASS |
| 13 | 0059617A2 | 60 | 1 | 37 | 20 | 27.1 | 5.14 | PASS |
| 14 | 0059599A2 | 60 | 1 | 48 | 23 | 36.2 | 5.85 | PASS |
| 15 | 0059616A2 | 40 | 1 | 45 | 24 | 27.8 | 5.29 | PASS |

Selection #1 0059071A2 — Best overall ¦ #6 0059391A2 — Best 2-stack ¦ #11 0059729A2 — Best 1-stack

Step 13.2 — Selected Core and Stacking Parameters

Core: Edge 0059071A2  |  OD = 33.66 mm, ID = 19.46 mm, HT = 11.43 mm (uncoated)

| AL(single) | 61 nH/T² ± 8% |
| --- | --- |
| Ae(single) | 66 mm² |
| Wa(single) | 297 mm² |
| Stacks | 3 |
| AL,total,min | 168 nH/T² |
| AL,total,nom | 183 nH/T² |
| AL,total,max | 198 nH/T² |
| Ae,total | 197 mm² |
| Wa,total | 891 mm² |

Step 13.3 — Number of Turns

Step 13.3.1 — Selected turns

N = 47 turns

Step 13.3.2 — No-load inductance (0 A) using AL,total

L0 = AL,total · N² L0,min = (168 nH/T²) · (47²) = 371.112 µH L0,nom = (183 nH/T²) · (47²) = 404.247 µH L0,max = (198 nH/T²) · (47²) = 437.382 µH

Step 13.3.3 — Required inductance retention at full-load bias

kreq = Lϕ,target / L0 kreq,min(AL) = 240.000 / 371.112 = 0.6467 kreq,nom(AL) = 240.000 / 404.247 = 0.5937 kreq,max(AL) = 240.000 / 437.382 = 0.5487 Bias (A·T) at low line full load (crest average): A·T = N · Iϕ,avg@crest A·T = 47 · 14.1522 = 665.15 A·T

Step 13.4 — Inductance at Full Load vs Input Voltage (Table and Plot)

Method (engineering estimate):

1) Use operating-point table to obtain Ipk,line at crest for each Vin,rms.

2) Compute per-phase crest-average DC current: Iϕ,avg@crest = Ipk,line / 2.

3) Compute bias in ampere-turns: A·T = N · Iϕ,avg@crest.

4) Apply an engineering retention model kbias = f(A·T) and compute Lfull = L0 · kbias.

Note kbias model provides a consistent inductance trend vs Vin for the chosen N and stacked core.

| Vin,rms (Vac) | Ipk,line (A) | Iϕ,avg@crest (A) | A·T (A·T) | kbias (—) | Lfull,min (µH) | Lfull,nom (µH) | Lfull,max (µH) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 90.0 | 28.3044 | 14.1522 | 665.2 | 0.5784 | 214.7 | 233.8 | 253.0 |
| 110.0 | 22.9180 | 11.4590 | 538.6 | 0.6766 | 251.1 | 273.5 | 295.9 |
| 120.0 | 20.7925 | 10.3963 | 488.6 | 0.7175 | 266.3 | 290.1 | 313.8 |
| 132.0 | 18.7178 | 9.3589 | 439.9 | 0.7580 | 281.3 | 306.4 | 331.5 |
| 180.0 | 29.6391 | 14.8196 | 696.5 | 0.5558 | 206.3 | 224.7 | 243.1 |
| 200.0 | 26.4150 | 13.2075 | 620.8 | 0.6118 | 227.0 | 247.3 | 267.6 |
| 220.0 | 23.9980 | 11.9990 | 564.0 | 0.6561 | 243.5 | 265.2 | 287.0 |
| 230.0 | 22.8873 | 11.4436 | 537.9 | 0.6772 | 251.3 | 273.7 | 296.2 |
| 264.0 | 20.4617 | 10.2308 | 480.8 | 0.7240 | 268.7 | 292.7 | 316.7 |

Plot — Inductance at full load versus input voltage.

Step 13.5 — Flux Density Swing at 0 A and Full Load (Crest Condition)

Step 13.5.1 — AC flux swing from applied volt-seconds

∆Bpp = Vin,pk · Dpk / (N · Ae,total · fsw) ∆Bpp = 127.2792 · 0.6767 / (47 · 197e-6 · 70000) ∆Bpp = 0.133027 T Bac,pk = ∆Bpp / 2 Bac,pk = 0.066513 T

Step 13.5.2 — DC flux density at full load (using Lϕ,target)

Bdc = Lϕ,target · Iϕ,avg@crest / (N · Ae,total) Bdc = 2.400000e-04 · 14.1522 / (47 · 1.968000e-04) Bdc = 0.367208 T

Step 13.5.3 — Flux swing range

No-load (I ≈ 0 A): Bmin,NL = −0.066513 T Bmax,NL = +0.066513 T Full-load (crest average current): Bmin,FL = 0.300695 T Bmax,FL = 0.433722 T

Step 13.6 — Winding Fill Factor

Step 13.6.1 — Selected wire

| Wire designation | MEW-AWG14 |
| --- | --- |
| Construction | MEW-AWG14 |
| Winding style | Bifilar (×2) |
| OD (per conductor) | 1.68 mm |
| Cu area per conductor (Acu,1) | 2.0809 mm² |
| Cu area per turn (npar × Acu,1) | 4.1618 mm² |
| Req(20°C) parallel bundle | 0.004135 Ω/m |

Step 13.6.2 — Copper fill factor

Acu,total = N · (npar · Acu,1) = 47 · (2 · 2.0809) = 47 · 4.1618 = 195.60 mm² FFcu = Acu,total / Wa,total = 195.60 / 891 = 0.2195 (22.0%)

Step 13.7 — Loss Calculation (Copper + Core): Low Line

Step 13.7.1 — Copper length

Dmean = (33.66 + 19.46)/2 = 26.560 mm MLT = π · Dmean = 83.4407 mm/turn Icu = N · MLT = 3.921713 m

Step 13.7.2 — DCR (25°C and 100°C)

R′(T) = R′(20°C) · [1 + α · (T − 20)] R′(20°C) = 0.004135 Ω/m DCR(25°C) = 16.5349 mΩ DCR(100°C) = 21.3147 mΩ

Step 13.7.3 — Copper loss at low line using Iϕ,rms

Pcu = Iϕ,rms² · DCR Iϕ,rms = 10.0700 A Pcu(25°C) = (10.0700)² · 16.5349 mΩ = 1.6767 W Pcu(100°C) = (10.0700)² · 21.3147 mΩ = 2.1614 W

Step 13.7.4 — Core loss estimate (anchored to datasheet reference)

| Reference | Pv,ref = 0.525 W/cm³ at (100 kHz, 0.1 T) |
| --- | --- |
| Bac,pk (90 Vac crest) | 0.066513 T |
| Vcore,total | 16.020000 cm³ |
| Pv | ≈ 0.120364 W/cm³ (estimate) |
| Pcore | ≈ 1.928235 W |

Step 13.7.5 — Total loss (low line)

Ptotal = Pcu + Pcore Ptotal(25°C) = 1.6767 + 1.9282 = 3.6050 W Ptotal(100°C) = 2.1614 + 1.9282 = 4.0896 W

Step 13.8 — Inductor Loss vs Input Voltage (Nominal AL, Per-phase, 25°C)

Assumptions for this sweep: 1) Nominal AL only (used for the selected turns N). 2) DCR uses 25°C winding resistance (constant vs Vin). 3) Copper loss uses Iϕ,rms from operating-point table for each Vin. 4) Core loss uses Bac,pk computed from Vin,pk and D@crest, anchored to datasheet loss density reference. 5) All losses below are per phase.

| Vin,rms (Vac) | Vin,pk (V) | D@crest (—) | Iϕ,rms (A) | Bac,pk (T) | Pcu (W) | Pcore (W) | Ptotal (W) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 90.0 | 127.279 | 0.6767 | 10.0700 | 0.06651 | 1.677 | 1.928 | 3.605 |
| 110.0 | 155.563 | 0.6049 | 8.2700 | 0.07266 | 1.131 | 2.363 | 3.494 |
| 120.0 | 169.706 | 0.5689 | 7.5400 | 0.07456 | 0.940 | 2.508 | 3.448 |
| 132.0 | 186.676 | 0.5258 | 6.8300 | 0.07580 | 0.771 | 2.605 | 3.376 |
| 180.0 | 254.558 | 0.3534 | 10.5900 | 0.06947 | 1.854 | 2.131 | 3.986 |
| 200.0 | 282.843 | 0.2816 | 9.4400 | 0.06150 | 1.473 | 1.610 | 3.084 |
| 220.0 | 311.127 | 0.2097 | 8.5600 | 0.05039 | 1.212 | 1.018 | 2.230 |
| 230.0 | 325.269 | 0.1738 | 8.1500 | 0.04366 | 1.098 | 0.732 | 1.831 |
| 264.0 | 373.352 | 0.0517 | 7.2400 | 0.01490 | 0.867 | 0.062 | 0.929 |

Plot — Inductor loss versus input voltage at 25°C.

Step 13.9 — Inductor Loss vs Input Voltage (Nominal AL, Per-phase, 100°C)

Assumptions for this sweep: 1) Nominal AL only (used for the selected turns N). 2) DCR uses 100°C winding resistance (constant vs Vin). 3) Copper loss uses Iϕ,rms from operating-point table for each Vin. 4) Core loss uses Bac,pk computed from Vin,pk and D@crest, anchored to datasheet loss density reference. 5) All losses below are per phase.

| Vin,rms (Vac) | Vin,pk (V) | D@crest (—) | Iϕ,rms (A) | Bac,pk (T) | Pcu (W) | Pcore (W) | Ptotal (W) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 90.0 | 127.279 | 0.6767 | 10.0700 | 0.06651 | 2.161 | 1.928 | 4.090 |
| 110.0 | 155.563 | 0.6049 | 8.2700 | 0.07266 | 1.458 | 2.363 | 3.821 |
| 120.0 | 169.706 | 0.5689 | 7.5400 | 0.07456 | 1.212 | 2.508 | 3.719 |
| 132.0 | 186.676 | 0.5258 | 6.8300 | 0.07580 | 0.994 | 2.605 | 3.599 |
| 180.0 | 254.558 | 0.3534 | 10.5900 | 0.06947 | 2.390 | 2.131 | 4.522 |
| 200.0 | 282.843 | 0.2816 | 9.4400 | 0.06150 | 1.899 | 1.610 | 3.510 |
| 220.0 | 311.127 | 0.2097 | 8.5600 | 0.05039 | 1.562 | 1.018 | 2.580 |
| 230.0 | 325.269 | 0.1738 | 8.1500 | 0.04366 | 1.416 | 0.732 | 2.148 |
| 264.0 | 373.352 | 0.0517 | 7.2400 | 0.01490 | 1.117 | 0.062 | 1.179 |

Plot — Inductor loss versus input voltage at 100°C.

Step 13.10 — Summary Table

| Core | 0059071A2 (3-stack) |
| --- | --- |
| Turns | 47 |
| AL,total (min/nom/max) | 168 / 183 / 198 nH/T² |
| L0 (0A) (min/nom/max) | 371.1 / 404.2 / 437.4 µH |
| kreq (min/nom/max) | 0.6467 / 0.5937 / 0.5487 |
| Ae,total | 197 mm² |
| Wa,total | 891 mm² |
| Copper fill factor (FFcu) | 22.0% |
| Bac,pk @ 90 Vac crest | 0.0665 T |
| Bdc @ 90 Vac full load | 0.3672 T |
| Flux range @ 90 Vac full load | 0.3007 to 0.4337 T |
| Wire | MEW-AWG14 |
| Copper length | 3.9217 m |
| DCR @ 25°C | 16.53 mΩ |
| Pcu @ 90Vac @25°C | 1.677 W |
| Pcore @ 90Vac (estimate) | 1.928 W |
| Ptotal @ 90Vac @25°C (estimate) | 3.605 W |

Step 14 — Time-Domain Flux and Core-Loss Modeling

Across Half Line Cycle — Detailed Pcore(t) Breakdown

This section computes Pcore(t) across the rectified half line cycle, following the time-domain modeling concept: model vs time/angle, then integrate. The core-loss model is anchored to the crest-point Pcore values from Step 13.8.

Step 14.1 — Governing equations

Vin(t) = Vpk · sin(2π · fline · t), t ∈ [0, 1/(2fline)] Vpk = √2 · Vac,rms D(t) = 1 − Vin(t) / Vbus ∆Bpp(t) = Vin(t) · D(t) / (N · Ae · fsw) Bac,pk(t) = ∆Bpp(t) / 2 ⇒ Bac,pk(t) = Vin(t) · D(t) / (2 · N · Ae · fsw)

Step 14.2 — Worked example: Bac,pk(t) for Vin = 90 Vac

| Given | Vac,rms = 90 V Vbus = 394 V |
| --- | --- |
| Given | fline = 60 Hz fsw = 70 kHz |
| Given | N = 47 turns Ae = 197 mm² |
| Compute | Vpk = √2 · 90 = 127.279 V |
| Compute | Thalf = 1/(2·60) = 8.333 ms |
| Compute | Vin(t) = Vpk · sin(2π·fline·t) |
| Compute | D(t) = 1 − Vin(t)/Vbus |
| Compute | Bac,pk(t) = Vin(t)·D(t)/(2·N·Ae·fsw) |
| Quantity | Value (90 Vac) |
| Peak Bac,pk(t) | 0.06651 T |
| Time at peak | 4.158 ms |
| Bac,pk at crest (θ=90°) | 0.06651 T |

Figure 14.1 — Bac,pk(t) over half line cycle at 90 Vac (per inductor).

Step 14.3 — Core-loss model used to compute Pcore(t)

A power-law model is used: Pcore = k · (Bac,pk)^n. Constants k and n are identified by fitting a log-log line to the crest-point data (Bac,pk@crest, Pcore@crest) across all input voltages.

Pcore(B) = k · Bⁿ ln(Pcore) = ln(k) + n · ln(B) Fit inputs: (Bi, Pi) = crest-point values from Step 13.8 Fit result: n = 2.3000 Fit result: k = 982.81 (units: Pcore in W when B in T)

| Vac,rms | Bac,pk@crest (T) | Pcore@crest (W) |
| --- | --- | --- |
| 90.0 | 0.06651 | 1.928 |
| 110.0 | 0.07266 | 2.363 |
| 120.0 | 0.07456 | 2.508 |
| 132.0 | 0.07580 | 2.605 |
| 180.0 | 0.06947 | 2.131 |
| 200.0 | 0.06150 | 1.610 |
| 220.0 | 0.05039 | 1.018 |
| 230.0 | 0.04366 | 0.732 |
| 264.0 | 0.01490 | 0.062 |

Validation Using Bac,pk@crest = 0.04366 T (230 Vac), model predicts Pcore = 0.732 W, matching crest-point value.

Step 14.4 — Constructing Pcore(t) from Bac,pk(t) and computing average loss

• Compute Bac,pk(t) using Step 14.1 equations.

• Compute Pcore(t) = k · (Bac,pk(t))^n

• Average over half-cycle: Pcore,avg = (1/Thalf) · ∫0^Thalf Pcore(t) dt

• Peak over half-cycle: Pcore,pk = max(Pcore(t))

Figure 14.2 — Pcore(t) over half line cycle at 230 Vac (per inductor), with average line.

Numerical summary 230 Vac numerical summary: Pcore,avg = 1.483 W; Pcore,pk = 2.621 W at t = 1.720 ms, where Vin ≈ 196.5 V and D ≈ 0.501.

Step 14.5 — Comparative waveforms and summary metrics for all Vac

Bac,pk(t) overlay for all input voltages:

Figure 14.3 — Bac,pk(t) overlay (per inductor).

Pcore(t) overlay for all input voltages:

Figure 14.4 — Pcore(t) overlay (per inductor).

Table 14.1 — Core-loss metrics derived from Pcore(t) across half line cycle (per inductor)

| Vac,rms (Vac) | Pcore,avg (W) | Pcore,pk (W) | t@peak (ms) | Vin@peak (V) | D@peak |
| --- | --- | --- | --- | --- | --- |
| 90 | 1.066 | 1.928 | 4.158 | 127.3 | 0.677 |
| 110 | 1.385 | 2.363 | 4.158 | 155.6 | 0.605 |
| 120 | 1.521 | 2.508 | 4.158 | 169.7 | 0.569 |
| 132 | 1.658 | 2.605 | 4.158 | 186.7 | 0.526 |
| 180 | 1.841 | 2.621 | 5.995 | 196.4 | 0.501 |
| 200 | 1.748 | 2.621 | 2.037 | 196.5 | 0.501 |
| 220 | 1.582 | 2.621 | 1.820 | 197.1 | 0.499 |
| 230 | 1.483 | 2.621 | 1.720 | 196.5 | 0.501 |
| 264 | 1.143 | 2.621 | 1.470 | 196.4 | 0.501 |

Step 15 — Vout Capacitor Design

Step 15.1 — Design Inputs

| Vout | 394 Vdc |
| --- | --- |
| fline | 60 Hz |
| Vdc,ripple | 20 V pk-pk |
| Vdc,min (hold-up floor) | 290 V |
| thold | 20 ms |
| Vout,max (transient) | 433 V |

Step 15.2 — Output Capacitance for Hold-up Time

Choldup = 2 · Pout · thold / (Vout² − Vdc,min²) Worst-case (180 Vac): Choldup = 2 · 3600 · 0.020 / (394² − 290²) = 2031.0 µF Low-line (90 Vac): Choldup = 2 · 1700 · 0.020 / (394² − 290²) = 959.1 µF

Step 15.3 — Output Capacitance for Voltage Ripple

Cripple = Pout / (2π · fline · η · Vout · Vdc,ripple) Worst-case (180 Vac): Cripple = 3600 / (2π · 60 · 0.965 · 394 · 20) = 1256.7 µF Low-line (90 Vac): Cripple = 1700 / (2π · 60 · 0.945 · 394 · 20) = 606.0 µF

Step 15.4 — Required Capacitance

Crequired = max(Choldup,worst, Choldup,low, Cripple,worst, Cripple,low) = 2031 µF Governing: C_holdup (worst-case)

Step 15.5 — Voltage Rating Selection

Vmin,rating = max(Vout × 1.12, Vout,max) = max(394 × 1.12, 433) = 440.9 V Selected rating: 450 V (next standard above 440.9 V)

Step 15.6 — Capacitor RMS Current: All 9 Operating Points

Idc = Pout / (η · Vout) ILF = Idc / √2 IHF = √(Idc² · 16Vout / (6Vin,pk · 2π) − ILF²) Itotal = √(ILF² + IHF²)

| Vin (Vac) | Pout (W) | η | Idc (A) | ILF (A) | IHF (A) | Itotal (A) |
| --- | --- | --- | --- | --- | --- | --- |
| 90 | 1700 | — | 4.569 | 3.231 | 4.119 | 5.235 |
| 110 | 1700 | — | 4.521 | 3.197 | 3.426 | 4.686 |
| 120 | 1700 | — | 4.475 | 3.164 | 3.115 | 4.440 |
| 132 | 1700 | — | 4.429 | 3.132 | 2.784 | 4.190 |
| 180 | 3600 | — | 9.476 | 6.700 | 3.747 | 7.677 |
| 200 | 3600 | — | 9.378 | 6.632 | 2.825 | 7.208 |
| 220 | 3600 | — | 9.283 | 6.564 | 1.787 | 6.803 |
| 230 | 3600 | — | 9.255 | 6.544 | 1.083 | 6.633 |
| 264 | 3600 | — | 9.236 | 6.531 | 0.000 | 6.531 |

Legend Blue row = worst-case (180 Vac) ¦ Green row = low-line (90 Vac)

Step 15.7 — Selected Capacitor Specifications

| Part / Value | Qty | Voltage (V) | ESR each (mΩ) | Irated (A) | Temp (°C) |
| --- | --- | --- | --- | --- | --- |
| 470 µF | 5 | 450 | 470.2 | 2.79 | 105 |

Selected bank Supplier: Panasonic ¦ Series: EEUFC — Long Life 105°C ¦ Ctotal = 2350 µF ¦ ESRpar = 94 mΩ ¦ Irated/cap = 2.79 A

Step 15.8 — Verified Performance

| Metric | Worst-case (180 Vac) | Low-line (90 Vac) |
| --- | --- | --- |
| Ctotal (µF) | 2350 | — |
| Margin over Creq | 15.7% | — |
| ESRparallel (mΩ) | 94 | — |
| Vripple,pp (V) | 10.696 | 5.158 |
| tholdup (ms) | 23.1 | 49 |
| Irms per cap (A) | 1.535 | 1.047 |
| Irated per cap (A) | 2.79 | 2.79 |
| Ripple current check | PASS | PASS |
| VESR,pk (V) | 0.722 | 0.492 |

Step 15.9 — Power Dissipation & Temperature Rise: All 9 Operating Points

Temp rating: 105°C  |  Rth,ca = 15 °C/W  |  Tamb = 50°C  |  I per cap = Itotal / X (X = number of caps)

| Vin (Vac) | Pout (W) | Itotal (A) | I/cap (A) | Irated (A) | Pdiss (W) | ∆T (°C) | Tcap (°C) | Vrip,pp (V) | Ripple Pass |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 90 | 1700 | 5.235 | 1.047 | 2.79 | 0.103 | 1.5 | 51.5 | 5.16 | PASS |
| 110 | 1700 | 4.686 | 0.937 | 2.79 | 0.083 | 1.2 | 51.2 | 5.10 | PASS |
| 120 | 1700 | 4.440 | 0.888 | 2.79 | 0.074 | 1.1 | 51.1 | 5.05 | PASS |
| 132 | 1700 | 4.190 | 0.838 | 2.79 | 0.066 | 1.0 | 51.0 | 5.00 | PASS |
| 180 | 3600 | 7.677 | 1.535 | 2.79 | 0.222 | 3.3 | 53.3 | 10.70 | PASS |
| 200 | 3600 | 7.208 | 1.442 | 2.79 | 0.195 | 2.9 | 52.9 | 10.59 | PASS |
| 220 | 3600 | 6.803 | 1.361 | 2.79 | 0.174 | 2.6 | 52.6 | 10.48 | PASS |
| 230 | 3600 | 6.633 | 1.327 | 2.79 | 0.166 | 2.5 | 52.5 | 10.45 | PASS |
| 264 | 3600 | 6.531 | 1.306 | 2.79 | 0.160 | 2.4 | 52.4 | 10.43 | PASS |

Result Worst-case Tcap = 53.3°C (rating 105°C) ¦ Ripple current: PASS

Step 15.10 — Summary

| Parameter | Worst-case (180 Vac) | Low-line (90 Vac) |
| --- | --- | --- |
| Crequired (µF) | 2031 | — |
| Ctotal selected (µF) | 2350 | — |
| Margin (%) | 15.7% | — |
| Voltage rating (V) | 450 | — |
| ESRparallel (mΩ) | 94 | — |
| Vout ripple pk-pk (V) | 10.696 | 5.158 |
| Hold-up time (ms) | 23.1 | 49 |
| Irms,total (A) | 7.677 | 5.235 |
| Irms per cap (A) | 1.535 | 1.047 |
| Irated per cap (A) | 2.79 | 2.79 |
| Ripple current check | PASS | PASS |