Boost Inductor Magnetic Design — Full Report

EDGE Core 0059392A2 (2-Stack) · 2-Channel Interleaved CCM PFC · Per-Phase Analysis

V_out = 390 Vdc · f_sw = 70 kHz · f_line = 60 Hz · N = 32 (AL-nominal) · validated EDGE 60µ material model · iGSE cycle-averaged losses

# Step 1) Reference Operating Point and Design Targets

## Step 1.1) Converter parameters

| Parameter | Value |
| --- | --- |
| Output voltage V_out | 390 Vdc |
| Switching frequency f_sw | 70 kHz |
| Line frequency | 60 Hz |
| Topology | 2-channel interleaved CCM boost PFC |
| Input range | 90 – 264 Vac (two power corners: 90–132 / 180–264) |
| Target inductance Lφ (low line, full load) | 235 µH per phase |

## Step 1.2) Per-phase operating currents (converter Table A)

Iφ,avg@crest = I_pk,line / 2        Iφ,rms includes switching ripple

| V_in (Vac) | V_in,pk (V) | I_pk,line (A) | Iφ,avg@crest (A) | Iφ,rms (A) | D@crest |
| --- | --- | --- | --- | --- | --- |
| 90 | 127.3 | 28.30 | 14.1522 | 10.1196 | 0.6736 |
| 110 | 155.6 | 22.92 | 11.4590 | 8.2672 | 0.6011 |
| 120 | 169.7 | 20.79 | 10.3963 | 7.5413 | 0.5649 |
| 132 | 186.7 | 18.72 | 9.3589 | 6.8346 | 0.5213 |
| 180 | 254.6 | 29.64 | 14.8195 | 10.5932 | 0.3473 |
| 200 | 282.8 | 26.41 | 13.2075 | 9.4382 | 0.2748 |
| 220 | 311.1 | 24.00 | 11.9990 | 8.5561 | 0.2022 |
| 230 | 325.3 | 22.89 | 11.4436 | 8.1472 | 0.1660 |
| 264 | 373.4 | 20.46 | 10.2309 | 7.2397 | 0.0427 |

Note: the 180–264 V rows carry higher currents than 90–132 V — the two ranges are distinct power corners. No interpolation is performed between 132 and 180 Vac anywhere in this report.

# Step 2) Methodology and Corrections Incorporated

This report supersedes the original 3-stack 0059894A2 calculation. Corrections identified and applied during design review:

| # | Correction | Effect |
| --- | --- | --- |
| 1 | Window area does not stack: Wa is the bore (156 mm² small core / 355 mm² this core), not Σ per-core Wa | Original 3-stack winding was physically unwindable; drove core change |
| 2 | Cross-section corrected (Ae = 153 mm²/core; original used wrong 231 mm² total) | Flux density and core loss corrected |
| 3 | MLT is the winding length/turn 2·(w + S·HT)+build, not π·D_mean (ring path) | Copper length +~30% vs original |
| 4 | Cycle-averaged time-domain core loss replaces crest-point value | Crest method erred +83% (90 V) to −97% (264 V) |
| 5 | Validated EDGE 60µ coefficients replace circular k/n curve fit | Material-anchored loss and bias models |
| 6 | True iGSE triangular-ripple correction F(D) applied | Core loss ~5% below sinusoidal-Steinmetz assumption |
| 7 | Explicit HF-ripple copper term I_hf²·R_ac | Quantified (≈8 mW) instead of assumed |
| 8 | SPG bias-loss adder NOT applied (powder core) | DC bias adds no core loss directly; bias acts via permeability roll-off only |

# Step 3) EDGE 60µ Material Model (Validated)

## Step 3.1) Core-loss density

P_L [mW/cm³] = a · B_ac,pk[T]^b · f[kHz]^c

b = 2.106    c = 1.444    (EDGE-family exponents from catalog fit)

a_max = 86.7   (anchored to part datasheet: 525 mW/cm³ max @ 0.1 T, 100 kHz)

a_typ = 67.4   (triangulated from 150 mW/cm³ typ @ 0.1 T, 50 kHz via exponent c)

Validation: a_max·0.1^2.106·100^1.444 = 525.0 mW/cm³ ✓ (reproduces datasheet anchor)

## Step 3.2) Permeability vs DC bias

%µi = 1 / (0.01 + 9.202×10⁻¹⁰ · H^3.044),   H in Oe

Validation: H=130 Oe → 80.0% ✓ ;  H=205 Oe → 50.0% ✓  (matches catalog spec points)

## Step 3.3) iGSE triangular-ripple correction

F(D) = 2^c · [D^(1−c) + (1−D)^(1−c)] / [(2π)^(c−1) · ∫₀^2π|cosθ|^c dθ],   ∫ = 3.5435

| D | 0.347 (180 V crest) | 0.5 (ripple max) | 0.674 (90 V crest) |
| --- | --- | --- | --- |
| F(D) | 0.953 | 0.924 | 0.963 |

Figure 3.1 — EDGE 60µ permeability vs DC bias, validated fit with operating point.

Figure 3.2 — EDGE core-loss density family at 50/70/100 kHz with operating flux marked.

# Step 4) Selected Core and Stacking Parameters — 0059392A2 × 2

| Quantity | Single core | 2-Stack | Stacks? |
| --- | --- | --- | --- |
| AL nominal (±8%) | 122 nH/T² | 244 nH/T² (224.5–263.5) | ×2 |
| Cross-section Ae | 153 mm² | 306 mm² | ×2 |
| Path length Le | 94 mm | 94 mm | unchanged |
| Window area Wa (bore) | 355 mm² | 355 mm² | unchanged |
| Volume Ve | 14.6 cm³ | 29.2 cm³ | ×2 |
| Dimensions (coated) | OD 40.94 / ID 21.27 / HT 17.89 mm | stack height 35.78 mm | — |

Figure 4.1 — 3D model of the 2-stack EDGE toroid with N=32 dual-litz winding (green epoxy coat per datasheet).

# Step 5) Number of Turns — N = 32 (AL-Nominal Sizing)

## Step 5.1) Governing equations

L₀ = AL,total · N²

H[Oe] = 0.4π · N · Iφ,avg@crest / Le[cm]

L_full = k(H) · L₀,   k(H) from Step 3.2

## Step 5.2) Worked example — 90 Vac full load

L₀,nom = 244 nH × 32² = 249.9 µH    (min 229.9 / max 269.8 µH)

A·T = 32 × 14.1522 = 452.9    H = 0.4π·452.9/9.4 = 60.5 Oe

k = 0.9761    L_full = 0.9761 × 249.9 = 243.9 µH  ≥ 235 µH ✓

## Step 5.3) Inductance retention at full-load crest — all input voltages

| V_in (Vac) | Iφ,avg@crest (A) | A·T | H (Oe) | k(H) | L_full min (µH) | L_full nom (µH) | L_full max (µH) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 90 | 14.15 | 452.9 | 60.5 | 0.9761 | 224.4 | 243.9 | 263.4 |
| 110 | 11.46 | 366.7 | 49 | 0.9873 | 226.9 | 246.7 | 266.4 |
| 120 | 10.40 | 332.7 | 44.5 | 0.9905 | 227.7 | 247.5 | 267.3 |
| 132 | 9.36 | 299.5 | 40 | 0.9931 | 228.3 | 248.1 | 268 |
| 180 | 14.82 | 474.2 | 63.4 | 0.9726 | 223.6 | 243 | 262.5 |
| 200 | 13.21 | 422.6 | 56.5 | 0.9806 | 225.4 | 245 | 264.6 |
| 220 | 12.00 | 384 | 51.3 | 0.9854 | 226.5 | 246.2 | 265.9 |
| 230 | 11.44 | 366.2 | 49 | 0.9874 | 227 | 246.7 | 266.4 |
| 264 | 10.23 | 327.4 | 43.8 | 0.991 | 227.8 | 247.6 | 267.4 |

Tolerance note: sized on AL-nominal, the worst-bias AL−8% case gives ~224 µH (≈5% under target) — the accepted consequence of nominal-AL sizing. N=33 is the floor for 235 µH at AL-min. Bias retention stays ≥97% everywhere (EDGE 60µ knee is far away at 130/205 Oe).

# Step 6) Inductance at Full Load vs Input Voltage

Figure 6.1 — L_full vs V_in for AL min/nominal/max with 235 µH target.

# Step 7) Flux Density

## Step 7.1) Governing equations and worked example (90 Vac crest)

ΔB_pp(t) = V_in(t)·D(t) / (N·Ae·f_sw)        B_ac,pk(t) = ΔB_pp(t)/2

B_dc(t) = L(H(t))·iφ(t) / (N·Ae)        B_max(t) = B_dc + B_ac,pk

ΔB_pp = 127.279 × 0.6736 / (32 × 306e-6 × 70000) = 0.1251 T  →  B_ac,pk = 0.0625 T

## Step 7.2) Flux summary — all input voltages

| V_in (Vac) | D@crest | B_ac,pk@crest (T) | B_dc peak (T) | B_max peak (T) |
| --- | --- | --- | --- | --- |
| 90 | 0.674 | 0.0625 | 0.352 | 0.415 |
| 110 | 0.601 | 0.0682 | 0.289 | 0.357 |
| 120 | 0.565 | 0.0699 | 0.263 | 0.333 |
| 132 | 0.521 | 0.0710 | 0.237 | 0.308 |
| 180 | 0.347 | 0.0645 | 0.368 | 0.432 |
| 200 | 0.275 | 0.0567 | 0.330 | 0.387 |
| 220 | 0.202 | 0.0459 | 0.302 | 0.348 |
| 230 | 0.166 | 0.0394 | 0.288 | 0.328 |
| 264 | 0.043 | 0.0116 | 0.259 | 0.270 |

Peak flux ≤0.43 T against EDGE saturation ≈1 T (>2.3× margin). B_ac,pk maximizes where V_in = V_out/2 = 195 V (D = 0.5), reached twice per half-cycle at high line.

Figure 7.1 — B_ac,pk(t) over half line cycle, all input voltages (per inductor).

Figure 7.2 — DC-bias flux excursion with switching-ripple band (90 / 180 Vac).

# Step 8) Magnetizing Field H(t)

iφ(t) = Iφ,pk·sin(θ)        H(t)[Oe] = 0.4π·N·iφ(t)/Le[cm]

Figure 8.1 — H(t) over half line cycle, all input voltages. Peak 63.4 Oe (180 Vac) stays far below the 50% knee at 205 Oe.

# Step 9) Winding Design and Window Fit

## Step 9.1) Wire and copper path

| Quantity | Value |
| --- | --- |
| Wire | 2 × TRW(F)(L)-0.1 litz (0.1 mm × 200) in parallel, 3.14 mm² Cu, bundle OD 2.23 mm |
| MLT (2-stack) = 2·(9.835 + 2×17.89) + 3.8 | 95 mm/turn |
| Copper length = 32 × MLT | 3.041 m |
| DCR @20 / 25 / 100 °C | 19.2 / 19.58 / 25.26 mΩ |

## Step 9.2) Window fit and layer screening

| Metric | Value | Assessment |
| --- | --- | --- |
| Bare-copper fill | 28.3 % | healthy |
| Insulated-bundle fill (2 × 2.23 mm/turn) | 70.4 % | full but windable |
| Geometric layer capacities (shrinking circumference) | 26 / 20 / 14 / 8 | 4 layers used (64 passes) |
| Remaining feed hole | ≈ 3.46 mm | shuttle-limited — confirm with winding house |

Figure 9.1 — Window build screening (first-order packing, not a geometric winding solver).

# Step 10) Copper Loss

P_cu = Iφ,rms² · DCR(T) + I_hf,rms² · (R_ac − R_dc),   R_ac/R_dc ≈ 1.15 (litz)

Skin depth @70 kHz = 0.249 mm  vs strand Ø0.1 mm → per-strand skin factor 1.0005 (negligible)

| V_in (Vac) | Iφ,rms (A) | I_hf,rms (A) | P_cu @25 °C (W) | P_cu @100 °C (W) |
| --- | --- | --- | --- | --- |
| 90 | 10.120 | 1.09 | 2.009 | 2.591 |
| 110 | 8.267 | 1.22 | 1.343 | 1.732 |
| 120 | 7.541 | 1.27 | 1.118 | 1.442 |
| 132 | 6.835 | 1.31 | 0.920 | 1.186 |
| 180 | 10.593 | 1.39 | 2.203 | 2.841 |
| 200 | 9.438 | 1.35 | 1.750 | 2.257 |
| 220 | 8.556 | 1.28 | 1.438 | 1.855 |
| 230 | 8.147 | 1.24 | 1.304 | 1.682 |
| 264 | 7.240 | 1.09 | 1.030 | 1.328 |

# Step 11) Core Loss — Time-Domain iGSE, Cycle-Averaged

## Step 11.1) Governing equations (one per line)

V_in(t) = V_pk·sin(2π·f_line·t),  t ∈ [0, 1/(2·f_line)]

D(t) = 1 − V_in(t)/V_out

B_ac,pk(t) = V_in(t)·D(t) / (2·N·Ae·f_sw)

P_core(t) = a · B_ac,pk(t)^b · (f_sw/1000)^c · F(D(t)) · Ve / 1000   [W]

P_core,avg = (1/T_half) ∫ P_core(t) dt        P_core,pk = max P_core(t)

## Step 11.2) Core-loss metrics — all input voltages (max-anchored; typ in parallel column)

| V_in (Vac) | P_core,avg typ (W) | P_core,avg max (W) | P_core,pk max (W) | t @pk (ms) | V_in @pk (V) | D @pk |
| --- | --- | --- | --- | --- | --- | --- |
| 90 | 1.533 | 1.973 | 3.282 | 4.17 | 127 | 0.674 |
| 110 | 1.882 | 2.421 | 3.831 | 4.17 | 156 | 0.601 |
| 120 | 2.027 | 2.608 | 4.005 | 4.17 | 170 | 0.565 |
| 132 | 2.169 | 2.792 | 4.115 | 4.17 | 187 | 0.521 |
| 180 | 2.364 | 3.041 | 4.128 | 2.31 | 195 | 0.500 |
| 200 | 2.268 | 2.918 | 4.128 | 6.31 | 195 | 0.500 |
| 220 | 2.087 | 2.686 | 4.128 | 1.80 | 195 | 0.500 |
| 230 | 1.973 | 2.538 | 4.128 | 1.71 | 195 | 0.500 |
| 264 | 1.540 | 1.981 | 4.128 | 1.46 | 195 | 0.500 |

At high line the peak occurs off-crest where V_in ≈ 195 V (D ≈ 0.5) — the double-hump signature. Using the crest value instead of the cycle average was the original report's largest core-loss error.

Figure 11.1 — P_core(t) (iGSE, max-anchored) over half line cycle, all input voltages.

# Step 12) Inductor Loss vs Input Voltage (Per Phase)

| V_in (Vac) | B_ac,pk@crest (T) | P_core typ (W) | P_core max (W) | P_cu 100°C (W) | P_total typ (W) | P_total max (W) | P_total max @25°C (W) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 90 | 0.0625 | 1.53 | 1.97 | 2.59 | 4.12 | 4.56 | 3.98 |
| 110 | 0.0682 | 1.88 | 2.42 | 1.73 | 3.61 | 4.15 | 3.76 |
| 120 | 0.0699 | 2.03 | 2.61 | 1.44 | 3.47 | 4.05 | 3.73 |
| 132 | 0.0710 | 2.17 | 2.79 | 1.19 | 3.36 | 3.98 | 3.71 |
| 180 | 0.0645 | 2.36 | 3.04 | 2.84 | 5.21 | 5.88 | 5.25 |
| 200 | 0.0567 | 2.27 | 2.92 | 2.26 | 4.53 | 5.17 | 4.67 |
| 220 | 0.0459 | 2.09 | 2.69 | 1.85 | 3.94 | 4.54 | 4.12 |
| 230 | 0.0394 | 1.97 | 2.54 | 1.68 | 3.65 | 4.22 | 3.84 |
| 264 | 0.0116 | 1.54 | 1.98 | 1.33 | 2.87 | 3.31 | 3.01 |

Worst case = 180 Vac: P_total = 5.21 W (typ) / 5.88 W (max) per inductor — copper (2.84 W) and core (3.04 W) co-dominant.

Figure 12.1 — Loss components vs input voltage (cycle-averaged, 100 °C copper).

# Step 13) Total Loss Over the Half Line Cycle

P(t) = P_core(t) + P_cu(t),    P_cu(t) = R_dc·iφ(t)² + R_ac·i_hf,rms(t)²

| V_in (Vac) | P_total,avg max (W) | P_total,pk (W) | t @pk (ms) |
| --- | --- | --- | --- |
| 90 | 4.564 | 8.402 | 4.17 |
| 110 | 4.153 | 7.219 | 4.17 |
| 120 | 4.050 | 6.809 | 4.17 |
| 132 | 3.978 | 6.403 | 4.17 |
| 180 | 5.883 | 9.078 | 4.17 |
| 200 | 5.175 | 7.211 | 4.17 |
| 220 | 4.541 | 5.978 | 2.39 |
| 230 | 4.221 | 5.620 | 6.21 |
| 264 | 3.310 | 5.002 | 1.64 |

The instantaneous total peaks at the line crest, copper-driven — e.g. 9.1 W at 180 Vac, 1.5× the 5.88 W cycle average. Thermal mass averages this 8.3 ms ripple (Step 14), but it matters for any fast hot-spot estimate.

Figure 13.1 — P_cu(t) at 100 °C, all input voltages.

Figure 13.2 — Total instantaneous loss P(t), all input voltages.

# Step 14) Thermal — Temperature Rise

ΔT[°C] = (P_total[mW] / SA[cm²])^0.833

SA = [π·OD_w·OH + (π/2)(OD_w²−hole²) + π·hole·OH]/100 = 102.2 cm²   (OD_w 45.3, hole 8, OH = 2×17.89+6.6 mm)

| V_in (Vac) | P_total max (W) | ΔT (°C) |
| --- | --- | --- |
| 90 | 4.56 | 23.7 |
| 110 | 4.15 | 21.9 |
| 120 | 4.05 | 21.4 |
| 132 | 3.98 | 21.1 |
| 180 | 5.88 | 29.3 |
| 200 | 5.17 | 26.3 |
| 220 | 4.54 | 23.6 |
| 230 | 4.22 | 22.2 |
| 264 | 3.31 | 18.1 |

Iterated steady state at Ta = 50 °C (180 Vac, max anchor): copper re-evaluated at the converged winding temperature gives P_total ≈ 5.69 W, ΔT ≈ 28.6 °C → winding ≈ 79 °C, leaving >120 °C margin to the 200 °C coating rating. The losses in Step 12 use a conservative fixed 100 °C winding.

Figure 14.1 — Estimated temperature rise vs input voltage.

# Step 15) Ripple Current and CCM/DCM Boundary

ΔI_pp(t) = V_in(t)·D(t) / (L(t)·f_sw)        CCM if iφ,avg(t) > ΔI_pp(t)/2

| V_in (Vac) | ΔI_pp max (A) | ΔI_pp @crest (A) | I_hf,rms (A) | CCM ratio @ZC |
| --- | --- | --- | --- | --- |
| 90 | 5.02 | 5.02 | 1.09 | 0.26 |
| 110 | 5.42 | 5.42 | 1.22 | 0.39 |
| 120 | 5.53 | 5.53 | 1.27 | 0.47 |
| 132 | 5.60 | 5.60 | 1.31 | 0.57 |
| 180 | 5.65 | 5.20 | 1.39 | 0.50 |
| 200 | 5.61 | 4.53 | 1.35 | 0.62 |
| 220 | 5.59 | 3.65 | 1.28 | 0.75 |
| 230 | 5.59 | 3.13 | 1.24 | 0.82 |
| 264 | 5.58 | 0.92 | 1.09 | 1.05 |

CCM ratio ≤ 0.51 everywhere at full load → CCM holds across the whole half-cycle, validating the core-loss waveform model. DCM appears near the zero-crossings only below ~50% load.

Figure 15.1 — ΔI_pp(t) over half line cycle, all input voltages.

# Step 16) Saturation and Inductance Margin

| Operating point | Current (A) | H (Oe) | L (µH) |
| --- | --- | --- | --- |
| Crest average (180 Vac) | 14.82 | 63.4 | 243.0 |
| Instantaneous peak (avg + ½ ripple) | 17.41 | 74.5 | ≈239 |
| 80 % inductance point | 30.4 | 130 | ≈195 |
| 50 % inductance point | 47.9 | 205 | ≈125 |

Peak flux 0.43 T vs ≈1 T saturation; 50% inductance not reached until 2.8× the operating peak current. EDGE soft saturation degrades gracefully under surge.

Figure 16.1 — L vs DC current with operating and margin points.

# Step 17) Interleaving Ripple Cancellation (System Level)

K(D) = (1−2D)/(1−D) for D≤0.5 ;  (2D−1)/D for D>0.5    (input ripple ÷ per-phase ripple, residual at 2·f_sw = 140 kHz)

Per-inductor flux and loss are NOT reduced by interleaving — every prior step analyzes one inductor at its full current. The benefit is input-side: best cancellation near D = 0.5 (~132 Vac crest).

Figure 17.1 — 2-phase cancellation: K(D) with labeled crest points (left) and K vs input voltage per regime (right).

# Step 18) Current Density and Window Utilization

| Metric | Value |
| --- | --- |
| Current density J (worst RMS / 3.14 mm²) | 3.37 A/mm² |
| Bare Cu / insulated fill | 28.3 % / 70.4 % |
| Layers (geometric screening) | 4 (26/20/14/8) |
| Feed hole remaining | ≈ 3.46 mm |

# Step 19) Energy Storage and Core Utilization

| Metric | Value |
| --- | --- |
| Stored energy ½·L·I_pk² | 37 mJ |
| Energy density | 1.27 mJ/cm³ |
| Area product available (Wa·Ae) | 10.86 cm⁴ |
| Area product required / utilization | ≈10.96 cm⁴ → ≈101 % (well-utilized) |

# Step 20) Converter-Level Roll-Up and Efficiency Impact

| V_in (Vac) | P_in (W) | Per-inductor typ (W) | Per-inductor max (W) | 2-inductor total max (W) | % of input |
| --- | --- | --- | --- | --- | --- |
| 90 | 1801 | 4.12 | 4.56 | 9.1 | 0.51 % |
| 110 | 1783 | 3.61 | 4.15 | 8.3 | 0.47 % |
| 120 | 1764 | 3.47 | 4.05 | 8.1 | 0.46 % |
| 132 | 1747 | 3.36 | 3.98 | 8.0 | 0.46 % |
| 180 | 3772 | 5.21 | 5.88 | 11.8 | 0.31 % |
| 200 | 3736 | 4.53 | 5.17 | 10.3 | 0.28 % |
| 220 | 3733 | 3.94 | 4.54 | 9.1 | 0.24 % |
| 230 | 3722 | 3.65 | 4.22 | 8.4 | 0.23 % |
| 264 | 3820 | 2.87 | 3.31 | 6.6 | 0.17 % |

# Step 21) Tolerance and Sensitivity

| Variable | Range | Effect |
| --- | --- | --- |
| Core AL | ±8 % | L 224–265 µH; ripple ∓8 %; core & copper loss unchanged (flux is volt-second set) |
| Winding temperature | 25 → 100 °C | P_cu +30 % (see Step 12 25 °C column) |
| Turns N | 31 / 32 / 33 | L ∝ N²; fewer N → higher B and core loss, lower copper |
| Core-loss anchor | typ → max | core loss +29 % (a = 67.4 → 86.7) — dominant open uncertainty |

# Step 22) Material Validity at the Operating Point

| Property | At operating point | Valid |
| --- | --- | --- |
| µ vs frequency | flat to ~3 MHz; f_sw = 70 kHz | ✓ |
| µ vs DC bias | 97 % at 63 Oe (knee 130/205 Oe) | ✓ |
| µ vs AC flux | ripple 0.06 T ≪ bias excursion | ✓ |
| µ vs temperature | EDGE best-in-class alloy-powder stability at 80–100 °C | ✓ |
| Loss model | reproduces datasheet 525 mW/cm³ anchor | ✓ |

# Step 23) Validation and Measurement Plan

| Verify | Method | Confirms |
| --- | --- | --- |
| L vs I (Steps 5, 16) | LCR with DC-bias fixture to ≥18 A | turns, retention, saturation margin |
| Core loss (Step 11) | two-winding B-H loop, phase de-skewed | iGSE model & anchor coefficient |
| Total loss (Step 12) | calorimetry — ground truth | core + copper, mechanism-independent |
| ΔT (Step 14) | winding thermocouple + IR at full load | thermal envelope model |
| Ripple/CCM (Step 15) | current probe across line cycle | CCM assumption |

# Step 24) Summary

| Item | Value |
| --- | --- |
| Core | EDGE 0059392A2 × 2 stack (60µ, AL 244 nH/T², Ae 306 mm², Ve 29.2 cm³) |
| Turns / wire | N = 32, dual TRW litz 0.1×200 (3.14 mm² Cu) |
| L₀ / L_full @ low line | 249.9 / 243.9 µH (target 235 µH ✓; AL−8 % gives ~224 µH) |
| Peak flux / saturation margin | 0.43 T / >2.3× to ≈1 T; 50 % L at 47.9 A (2.8× peak) |
| DCR @100 °C / window fill | 25.26 mΩ / 70.4 % insulated (4 layers, 3.46 mm feed hole) |
| Worst-case loss (180 Vac, 100 °C) | 5.21 W typ / 5.88 W max per inductor (instantaneous peak 9.1 W at crest) |
| Temperature rise | ≈29 °C → winding ≈79 °C @ 50 °C ambient (rating 200 °C) |
| CCM / interleaving | CCM across full half-cycle at full load; input ripple residual at 140 kHz |
| Open items | exact standard-EDGE 60µ loss coefficient (typ vs max band); winding-house confirmation of 4-layer / 3.5 mm feed-hole build; measured wound SA for thermal |

Model: validated EDGE 60µ coefficients (catalog/datasheet), true iGSE triangular-ripple correction, time-domain cycle-averaged losses, converter currents per supplied Table A. All figures and tables generated from a single consistent engine, cross-checked against the interactive studio tool (V5.1).