# Nomenclature

| Symbol | Description | Value / Units |
| --- | --- | --- |
| Lo | Datasheet base life at Tmax and rated ripple | 8000 h @ 105 °C |
| L | Estimated life in application | h / yr |
| Tmax | Maximum category temperature | 105 °C |
| Tamb | Capacitor ambient (2–5 cm from can) | 45 °C |
| Tc | Measured can surface temperature | °C (bench) |
| ΔTo | Rated core rise at rated ripple Io | 5 °C (snap-in) |
| ΔTj | Actual core rise from applied ripple | °C |
| α | Diameter coefficient (core-to-surface) | — (by Ø) |
| Io | Rated ripple current | 3.0 A @100 Hz |
| Ix_n / I | Measured ripple per component / converted | A rms |
| kf_n | Frequency coefficient (per component) | — |
| k | Product-type coefficient | 0.17 (snap-in) |
| kv | Product size constant (by can length) | 3.37 (60 mm) |
| V / Vo | Applied / rated capacitor voltage | 390 / 450 V |
| f(T), f(I), f(V) | Temperature, ripple, voltage life factors | × |

# Introduction — Why Electrolytic Capacitors Wear Out

An aluminium electrolytic capacitor ages because its liquid electrolyte slowly escapes through the rubber end-seal. As electrolyte is lost, capacitance falls and ESR rises until the part no longer meets spec — this is wear-out, not random failure. The escape rate is a chemical reaction, so it follows the Arrhenius law: it roughly doubles for every 10 °C rise in the temperature of the winding (the “core”). Everything in this report is ultimately about estimating that core temperature accurately and converting it to years of service.

THEORY   The core runs hotter than the ambient air for two reasons: the surrounding air temperature (Tamb), and self-heating from ripple current flowing through the ESR (ΔTj). The manufacturer model treats these two contributions with separate multipliers — f(T) for ambient and f(I) for ripple — rather than lumping them into one temperature. A third multiplier, f(V), credits the extra margin from operating below the rated voltage.

The complete manufacturer life equation is therefore a product of the datasheet base life and three dimensionless factors:

L  =  Lo · f(T) · f(I) · f(V)

The next nine steps establish the inputs, compute each factor for the selected bank, combine them, and compare the result with the simplified model used previously.

# Step 15 — Output Capacitor: Sizing, Selection and Lifetime

This step sizes the bulk output capacitor from the hold-up and ripple requirements (15.1–15.4), selects a specific part from the capacitor database (15.5), verifies the ripple currents and ripple voltage of the chosen bank (15.6–15.8), and then computes the service life of the selected part with the manufacturer endurance model (15.9–15.16).

Step 15.1 — Inputs

CONCEPT   The output capacitor must do two jobs at once: hold the bus up through a line dropout for t_hold, and keep the 2·f_line ripple voltage within spec. Each job sets a minimum capacitance; the larger governs.

| Quantity | Value |
| --- | --- |
| Output / bus voltage, V_out | 390 Vdc |
| Line frequency, f_line | 60 Hz |
| Ripple voltage spec (pk-pk) | 20 V |
| Minimum hold-up voltage, V_dc,min | 290 Vdc |
| Hold-up time, t_hold | 20 ms |
| Worst-case point | V_in,rms = 180 Vac, P_out = 3600 W, η = 0.965 |
| Low-line reference | V_in,rms = 90 Vac, P_out = 1700 W, η = 0.945 |

Decision   Operating point fixed at 390 V bus, 3.6 kW worst case. All sizing below is evaluated at the worst-case point and cross-checked at low line.

Step 15.2 — C_out from Hold-up

THEORY   During a line dropout the capacitor supplies the full load while its voltage falls from V_out to V_dc,min. Equating the stored-energy change to the energy delivered gives the minimum capacitance.

Step 1   Hold-up capacitance:

C_out,hold = (2 · P_out · t_hold) / (V_out² − V_dc,min²)

Step 2   Denominator:

V_out² − V_dc,min² = 152100 − 84100 = 68000

C_out,hold = 0.0021176 F = 2117.6 µF

Decision   C_out,hold = 2117.6 µF. This is the larger of the two demands, as shown next.

Step 15.3 — C_out from Ripple

THEORY   The load draws DC but the rectified line delivers power at twice the line frequency, so the capacitor sources/sinks a 2·f_line ripple. The capacitance must be large enough to keep that ripple voltage within the 20 V spec.

Step 1   Ripple capacitance:

C_out,ripple = P_out / (2π · f_line · η · V_out · V_ripple,pp)

C_out,ripple = 0.0012687 F = 1268.7 µF

Decision   C_out,ripple = 1268.7 µF. Smaller than the hold-up demand, so hold-up governs the sizing.

Step 15.4 — Required Capacitance and Bank Decision

Step 1   Required capacitance is the larger demand:

C_out,required = max(2117.6 , 1268.7) = 2117.6 µF

A single 450 V snap-in cannot supply both 2117.6 µF and the ~13 A rms ripple of Step 15.6, so the bus is built as a parallel bank. Using five 470 µF parts:

C_bank = 5 × 470 µF = 2350 µF    ( +11.0% margin over 2117.6 µF )

INSIGHT   Five parts is driven by ripple-current sharing, not capacitance alone: spreading 12.97 A across five cans keeps each one at well under half its rating, which (as Step 15.13 shows) is what gives the bank its very long life.

Decision   Selected C_out = 2350 µF (5 × 470 µF / 450 V in parallel). Carried into the ripple and life calculations below.

Step 15.5 — Capacitor Selection from Database

Voltage rating with derating: V_rated,min = 1.10 × V_bus = 1.10 × 390 = 429 V → select the 450 V class. Filtering the capacitor database for 470 µF / 450 V snap-in parts and ranking by ripple-current capability, published ESR and rated load-life, the selected part is:

| Parameter | Value | Parameter / Value |
| --- | --- | --- |
| Manufacturer / Series | KEMET / ALC40 | Rated life: 8000 h @ 105 °C |
| Mfr Part # | ALC40A471DH450 | Operating temp: −40 °C ~ +105 °C |
| Capacitance / voltage | 470 µF / 450 V | ESR: 0.232 Ω @ 100 Hz |
| Tolerance | ±20% | Impedance (HF): 0.138 Ω |
| Ripple — LF | 3.00 A @ 100 Hz | Ripple — HF: 5.82 A @ 10 kHz |
| Case / mounting | Ø35 × 62 mm, Snap-In | Lead spacing: 10.0 mm |
| Qty in bank | 5 (parallel) | RoHS: Yes |

Note  Selected for long 8000 h load-life, a published 100 Hz ESR (enables a direct ESR cross-check), and high ripple headroom. Every datasheet parameter above was read from the combined HV Al-Elec capacitor database.

Decision   Selected part: KEMET ALC40A471DH450, ×5. Datasheet parameters here are the fixed inputs for the life model (15.9 onward).

Step 15.6 — RMS Capacitor Currents

THEORY   The capacitor carries a low-frequency component (the 2·f_line rectifier ripple) and a high-frequency component (the switching ripple from the boost inductors). They add in quadrature to the total rms current the bank must handle.

Step 1   Total bank currents:

I_dc = P_out/(η·V_out)

I_Cout,LF = I_dc/√2

I_Cout,HF = √( I_dc²·(16·V_out)/(6·V_in,min·2π) − I_LF² )

I_total = √(I_LF² + I_HF²)

Worst-case:  I_LF = 6.764 A,  I_HF = 11.069 A,  I_total = 12.972 A

Low-line:    I_LF = 3.262 A,  I_HF = 5.338 A,   I_total = 6.256 A

Step 2   Per-capacitor share (÷5, equal parallel caps), worst-case:

I_LF,cap = 1.353 A    I_HF,cap = 2.214 A    I_total,cap = 2.594 A

Utilisation per cap:  LF 1.353/3.00 = 45%   HF 2.214/5.82 = 38%   → ample headroom

Decision   Per-cap ripple: 1.353 A LF + 2.214 A HF (45% / 38% of rating). These per-component currents feed the frequency conversion in Step 15.10.

Step 15.7 — Ripple Voltage with 2350 µF

Step 1   Bus ripple voltage on the selected bank:

V_ripple,pp = P_out / (2π · f_line · C_bank · η · V_out)

Worst-case V_ripple,pp = 10.80 Vpk-pk   (spec 20 V → PASS)

Low-line   V_ripple,pp = 5.21 Vpk-pk    (spec 20 V → PASS)

Decision   Ripple voltage 10.80 Vpp worst case — within the 20 V spec. The 2350 µF bank satisfies both hold-up and ripple-voltage requirements.

Step 15.8 — Sizing Summary

| Item | Worst-case (3.6 kW) | Low-line (1.7 kW) |
| --- | --- | --- |
| C_out from hold-up (20 ms, 390→290 V) | 2117.6 µF | 1000.0 µF |
| C_out from ripple (20 Vpk-pk) | 1268.7 µF | 611.8 µF |
| C_out required (max of both) | 2117.6 µF | 1000.0 µF |
| Selected C_out (5 × 470 µF) | 2350 µF | 2350 µF |
| V_ripple,pk-pk with 2350 µF | 10.80 V | 5.21 V |
| Bank ripple I_total | 12.972 A | 6.256 A |
| Per-cap I_total (÷5) | 2.594 A | 1.251 A |

Decision   Sizing complete: 5 × 470 µF / 450 V, C_bank = 2350 µF. The remainder of Step 15 computes the service life of this bank.

Step 15.9 — Lifetime: Overview, Inputs and Coefficients

THEORY   Aluminium-electrolytic life is governed by electrolyte loss through the end-seal — an Arrhenius process whose rate doubles per ~10 °C of core temperature. Three methods of increasing rigour are evaluated and compared: (1) a simplified Arrhenius estimate using the datasheet ESR, (2) the same Arrhenius form using a tan-δ-derived ESR, and (3) the full manufacturer model L = Lo·f(T)·f(I)·f(V).

CONCEPT   Methods 1 and 2 lump all heating into a single core temperature T_core = Tamb + ΔT, where ΔT = (I_LF²·ESR_LF + I_HF²·ESR_HF)·R_th, then apply one Arrhenius term and a cubic voltage law. Method 3 separates ambient and ripple heating and uses datasheet-anchored factors.

## 15.9.1  Common inputs

| Parameter | Symbol | Value |
| --- | --- | --- |
| Base life at Tmax | Lo | 8000 h |
| Max category temperature | Tmax | 105 °C |
| Capacitor ambient | Tamb | 45 °C |
| Per-cap ripple (from 15.6) | I_LF / I_HF | 1.353 / 2.214 A |
| Rated ripple (low / high freq.) | Io / Io,HF | 3.00 / 5.82 A |
| Applied / rated voltage | V / Vo | 390 / 450 V |
| Thermal resistance (Methods 1–2) | R_th | 10 °C/W (Ø35 mm) |
| Voltage exponent (Methods 1–2) | n | 3 |

## 15.9.2  Manufacturer-model coefficients (Method 3)

| Coefficient | Value | Source / rule |
| --- | --- | --- |
| Product-type k | 0.17 | Snap-in. (Lead 0.25, screw 0.) Sets curvature of f(I). |
| Rated core rise ΔTo | 5 °C | Snap-in “other types”; datasheet value preferred. |
| Freq. coeff. kf (120 Hz / 10 kHz) | 1.00 / 1.94 | kf(10 kHz) = Io,HF/Io = 5.82/3.00. |
| Size constant kv | 3.37 | Can length 60 mm row (62 mm uses 60 mm). |

Decision   Three methods, common operating point. Methods 1–2 (15.10–15.11) use R_th and ESR; Method 3 (15.12–15.15) uses the manufacturer coefficients.

Step 15.10 — Method 1: Simplified Arrhenius from Datasheet ESR

THEORY   The most direct estimate: heat the core with the datasheet ESR for the low-frequency term and the datasheet HF impedance for the switching term, raise the core above ambient by ΔT = P·R_th, and apply one Arrhenius term plus a cubic voltage derating.

Step 1   Self-heating power and core temperature (per cap):

P = I_LF²·ESR_LF + I_HF²·ESR_HF = 1.353²·0.232 + 2.214²·0.138 = 1.10 W

ΔT = P·R_th = 1.10 × 10 = 11.0 °C    T_core = 45 + 11.0 = 56.0 °C

Step 2   Arrhenius life and voltage derating:

L = Lo·2^((Tmax−T_core)/10)·(Vo/V)³ = 8000·2^((105−56.0)/10)·(450/390)³

= 8000 · 29.86 · 1.536

L (temp only) = 238,700 h (27.2 yr)    L (temp + voltage) = 366,700 h (41.9 yr)

Decision   Method 1 life = 41.9 yr (datasheet ESR). Realistic expectation; uses measured 100 Hz ESR and HF impedance.

Step 15.11 — Method 2: Simplified Arrhenius from tan-δ-Derived ESR

THEORY   When a part lists no ESR, it can be derived from the dissipation factor: ESR_LF = tan-δ/(2π·120·C). The same Arrhenius form is then applied. Using the worst-case catalogue tan-δ gives a conservative bound.

Step 1   tan-δ-derived ESR (tan-δ_max = 0.20 for the 450 V class):

ESR_LF = 0.20/(2π·120·470 µF) = 0.564 Ω    ESR_HF = 0.30·ESR_LF = 0.169 Ω

Step 2   Self-heating and core temperature:

P = 1.353²·0.564 + 2.214²·0.169 = 1.86 W    ΔT = 18.6 °C    T_core = 63.6 °C

Step 3   Arrhenius life and voltage derating:

L = 8000·2^((105−63.6)/10)·(450/390)³ = 8000·17.63·1.536

L (temp only) = 140,800 h (16.1 yr)    L (temp + voltage) = 216,300 h (24.7 yr)

INSIGHT   Using the typical tan-δ (0.55 × max = 0.11) instead of the worst-case max gives ESR_LF = 0.310 Ω, T_core = 55.2 °C and L (temp + V) ≈ 386,700 h (44.1 yr) — essentially matching Method 1. The max-tan-δ figure above is the conservative design bound; the typical value validates the datasheet ESR.

Decision   Method 2 life = 24.7 yr (worst-case tan-δ) / 44.1 yr (typical). Conservative bound for parts without a published ESR.

Step 15.12 — Method 3: Ripple Current Conversion to the Rated Frequency

THEORY   The manufacturer model first normalises every ripple component to the rated frequency using the part's frequency coefficients, combining them in quadrature — more accurate than a single ESR for all frequencies.

Step 1   Convert each component to the rated 100 Hz reference:

I = √( (I_LF/kf_LF)² + (I_HF/kf_HF)² ) = √( (1.353/1.00)² + (2.214/1.94)² )

I = 1.770 A rms (equivalent ripple at the rated frequency)

PITFALL   Do not apply the LF ESR to the HF current. The HF component is divided by its larger frequency coefficient (1.94) because the part tolerates more ripple there — skipping this over-states the heating by ~25%.

Decision   Converted ripple I = 1.770 A rms. Used for the datasheet-anchored core rise in Step 15.13.

Step 15.13 — Method 3: Core Rise ΔTj and Temperature Factor f(T)

THEORY   Method 3 anchors self-heating to the datasheet rated core rise ΔTo at the rated current (no R_th estimate), and credits the cool ambient with a separate temperature factor floored at 40 °C.

Step 1   Core rise from rated ΔTo and the I² law:

ΔTj = ΔTo·(I/Io)² = 5·(1.770/3.00)² = 1.74 °C   (T_core ≈ 46.7 °C)

Step 2   Temperature factor on ambient (40 °C floor):

f(T) = 2^((Tmax−Tamb)/10) = 2^((105−45)/10) = 2^6

f(T) = 64×

PITFALL   Use Tamb in f(T), not T_core — the ripple heating is handled separately by f(I). This is the key structural difference from Methods 1–2, which put the (small) self-heating into T_core.

Decision   ΔTj = 1.74 °C; f(T) = 64×. Bench cross-check: Tamb = Tc − ΔTj/α (α = diameter coefficient).

Step 15.14 — Method 3: Ripple Factor f(I) and Voltage Factor f(V)

Step 1   Ripple factor (snap-in k = 0.17):

f(I) = 2^( ΔTo/(10−k·ΔTo) − ΔTj/(10−k·ΔTj) ) = 2^( 5/9.15 − 1.74/9.70 ) = 2^0.367

f(I) = 1.29×

Step 2   Voltage factor (size constant kv = 3.37):

f(V) = min( kv , 5(kv−1)(1−V/Vo)+1 ) = min( 3.37 , 5·2.37·0.1333+1 ) = min(3.37, 2.58)

f(V) = 2.58×

PITFALL   The size-based f(V) = 2.58× is well above the (450/390)³ = 1.54× used by Methods 1–2 — a 67% difference that is the main reason Method 3 reads higher than the simplified methods.

Decision   f(I) = 1.29×, f(V) = 2.58×. Combined with f(T) in Step 15.15.

Step 15.15 — Method 3: Life and Pass/Fail

Step 1   Combine the factors:

L = Lo·f(T)·f(I)·f(V) = 8000 · 64 · 1.29 · 2.58 ≈ 1.70 × 10⁶ h

L ≈ 194 yr model output  →  capped at 15 yr design target  →  PASS

INSIGHT   The uncapped figure is physically meaningless — no electrolytic survives two centuries — but it shows the part is enormously de-rated. The honest deliverable is “exceeds the 15-year target with very large margin.”

Decision   Method 3 life > 15-year target — PASS, very large margin. Manufacturer-model result for the qualification record.

Step 15.16 — Three-Method Lifetime Comparison

The three methods are summarised below for one capacitor of the five-unit bank, worst case, 45 °C ambient. All three clear the 15-year design target; they differ in rigour and in how they treat ambient versus ripple heating.

| Method | Heating / ESR basis | Temp. treatment | Life (temp + V) |
| --- | --- | --- | --- |
| 1 — Datasheet ESR (Arrhenius) | ESR 0.232/0.138 Ω → ΔT 11.0 °C, T_core 56.0 °C | 2^((105−56)/10) = 29.9× | 366,700 h (41.9 yr) |
| 2 — tan-δ ESR, max (Arrhenius) | ESR 0.564/0.169 Ω → ΔT 18.6 °C, T_core 63.6 °C | 2^((105−63.6)/10) = 17.6× | 216,300 h (24.7 yr) |
| 2 — tan-δ ESR, typical (×0.55) | ESR 0.310 Ω → T_core 55.2 °C | 2^((105−55.2)/10) = 30.9× | 386,700 h (44.1 yr) |
| 3 — Manufacturer model | ΔTj 1.74 °C (datasheet ΔTo); f(I) 1.29× | f(T) = 64× (on ambient) | ~1.70×10⁶ h (≈194 → cap 15 yr) |

Note  Voltage factor: Methods 1–2 use (Vo/V)³ = 1.54×; Method 3 uses the size-based f(V) = 2.58×. This, plus f(T) on ambient (64×) versus 2^((Tmax−T_core)/10) on core (≈18–30×), explains why Method 3 reads highest.

INSIGHT   Read the spread as a confidence band, not a contradiction. Method 2 (worst-case tan-δ) is the conservative floor at ~25 yr; Method 1 (datasheet ESR) is the realistic expectation at ~42 yr; Method 3 (manufacturer) is the formal qualification method and shows the part is hugely de-rated. The lowest of the three already exceeds the 15-year target by 65%, so the bank is endorsed regardless of which method the reviewer prefers.

Decision   All three methods PASS the 15-year target (floor 24.7 yr). Adopt Method 3 (manufacturer model) for the qualification record; keep Methods 1–2 as cross-checks. The 5 × 470 µF ALC40 bank is endurance-qualified for the 390 V / 3.6 kW bus at 45 °C.