1. FAN9672 control architecture

FAN9672 uses two active loops:

Inner loop — current tracking

The current loop forces average inductor current to track the gain-modulator command.

FAN9672 equation:

ILRCS=IMORM⋅Gain2

For channel 1:

Gain2=1

For channel 2, Gain2depends on channel-management state. At full load with both channels enabled, use:

Gain2≈1

The FAN9672 datasheet states that the current command comes from the IAC/VEA/VLPK gain modulator and that the current loop regulates ILRCSto the current command.

Outer loop — voltage regulation

The voltage loop regulates FBPFC to the internal reference:

VFBPFC=2.5 V

The GMV amplifier output is VEA, which feeds the gain modulator.

2. Power-stage values from your 2-channel design

From the attached 2-channel PFC design:

VOUT=390 V
fSW=70 kHz
Lϕ=235 μH
COUT=2200 μF

The design report selects COUT=2200 μF, based on hold-up and ripple requirements.

For two phases:

Leq=Lϕ2
Leq=117.5 μH

13. Voltage-loop plant

AN4165 gives the low-frequency control-to-output plant for line-feedforward PFC as:

vPFCvVEA≈IOUT,TOT⋅KMAX5⋅sCOUT

where 5 V is the approximately linear VEA window.

Use:

POUT=3600W
VOUT=390V
IOUT,TOT=POUTVOUT
IOUT,TOT=3600390=9.23A

Assume:

KMAX=1.3
COUT=2200μF

Then the low-frequency plant magnitude is:

Gvo(s)=9.23⋅1.35⋅2200μF⋅s
Gvo(s)=1090s

Conceptually, this is an integrator: more VEAcommands more input current/power, which charges the bus capacitor.

14. Voltage-loop design target

You selected:

fCV=17Hz

This is below the 20 Hz hard guardrail we discussed, but it is above the conservative 1/10to 1/5line-frequency range from AN4165-style guidance. For 60 Hz:

6010=6Hz
605=12Hz

So:

17Hz is usable as an aggressive target, but it has more 120Hz ripple interaction risk

19. Gain modulator and IAC check

For universal input, FAN9672/FAN9673 uses:

RIAC=6MΩ

when VIR selects full-range input. AN4165 states that for universal input, RIAC=6MΩ, while high-line-only input uses 12MΩ.

At 264 Vac:

Vin,pk=373.35V
IAC,pk=373.356MΩ
IAC,pk=62.2μA

The FAN9672 datasheet lists the IAC multiplier linear range up to 65 µA.

So:

62.2μA<65μA

This is acceptable but close to the upper limit. Use accurate resistor tolerance.

20. Oscillator resistor check for 70 kHz

FAN9672 oscillator equation:

fOSC=8×108RRI

For:

fSW=70kHz
RRI=8×10870k
RRI=11.43kΩ

This falls inside the FAN9672 high-frequency range.

Use:

RRI=11.3kΩ or 11.5kΩ

. Why AN4165 eq. 31 and AND9925 eq. 11 differ

AN4165 eq. 31

AN4165 designs RCSfrom the maximum power limit per channel:

RCS=VLINE,min2⋅GMAX⋅RMRIAC⋅POUT,max,phase

AN4165 uses this as the first control-circuit design step and explains that RCSis selected considering the voltage-loop control window. It also says the maximum power limit is typically set to about 120–150% of full load.

This equation is easier and more practical when you know:

- minimum line voltage

- max power per channel

- RIAC

- RM

- GMAX

AND9925 eq. 11

AND9925 derives RCSfrom the gain-modulator equation:

VRM(t)=KRMIIAC,pk(VVEA-0.6)∣sin⁡ωt∣2KRLPKIIAC,pkRRLPK2

Then because VCStracks the current command VRM, AND9925 derives:

RCS=KRMRIAC(VVEA,max-0.6)8KRLPK2RRLPK2(PMAX/NCH)

AND9925 explains that KRM≈6000, VEAhas a 0.6–5.6 V linear range, and RCSshould set VEA,maxaround 4–5 V for stable operation.

2. Practical interpretation

AN4165 eq. 31 is a simplified design equation

It assumes:

- pre-assumed gain-modulator behavior

- GMAX≈2

- RM≈7.5kΩ

- design based on maximum allowed power per channel

AND9925 eq. 11 is more general

It explicitly includes:

- KRM

- KRLPK

- RRLPK

- chosen VEA,max

- number of channels

So AND9925 is better for understanding why the sense resistor controls the VEA operating window.

AN4165 is better for quick design.

3. Apply both to your design

Your design values:

VOUT=390V
POUT,LL=1700W
NCH=2
Pphase=17002=850W

Use 130% max-power margin:

PMAX,phase=1.3×850=1105W

Universal input:

RIAC=6MΩ

FAN9672/FAN9673 universal input uses RIAC=6MΩ, while high-line-only input uses 12MΩ.

4. AN4165 eq. 31 result

Use:

VLINE,min=90V
GMAX=2
RM=7.5kΩ
RIAC=6MΩ
PMAX,phase=1105W
RCS=902⋅2⋅75006×106⋅1105
RCS=18.3mΩ

So AN4165 gives:

RCS≈18mΩ

If instead the design uses 85 Vac brownout/minimum operating line:

RCS=852⋅2⋅75006×106⋅1105
RCS=16.3mΩ

So practical AN4165 range is:

16mΩ to 18mΩ

5. AND9925 eq. 11 result

Use:

KRM=6000
KRLPK=2.465
RRLPK=12.4kΩ
RIAC=6MΩ
PMAX/NCH=1105W

If we target:

VEA,max=4.5V

then:

RCS=6000⋅6M⋅(4.5-0.6)8⋅2.4652⋅(12.4k)2⋅1105
RCS=17.0mΩ

So AND9925 gives:

RCS≈17mΩ

If VEA,maxis varied:

| VEA,max | RCS |
| --- | --- |
| 4.0 V | 14.8 mΩ |
| 4.5 V | 17.0 mΩ |
| 5.0 V | 19.2 mΩ |
| 5.6 V | 21.8 mΩ |

AND9925 says VEA,maxis generally set around 4–5 V, so a practical target is:

15mΩ to 19mΩ

6. Why 7 mΩ is not a good final value

Earlier, using RCS=7mΩ, the LS resistor was:

RLS=143kΩ

But AN4165 says RLSshould be within approximately:

12kΩ≤RLS≤87kΩ

The LS/LPT equation is:

RLS=LPFC1.5×10-9⋅RCS⋅(VOUT/VFB)

For your design:

LPFC=235μH
VOUTVFB=3902.5=156

With RCS=7mΩ:

RLS=235μH1.5×10-9⋅0.007⋅156
RLS=143kΩ

That is outside the allowed range.

To force RLS≤87kΩ:

RCS≥235μH1.5×10-9⋅156⋅87k
RCS≥11.5mΩ

So the absolute practical lower bound is:

RCS,min≈11.5mΩ

7. Best solution

Combining all constraints:

| Method / constraint | Result |
| --- | --- |
| AN4165 eq. 31 at 90 Vac | 18.3 mΩ |
| AN4165 eq. 31 at 85 Vac | 16.3 mΩ |
| AND9925 eq. 11, VEA,max=4.0V | 14.8 mΩ |
| AND9925 eq. 11, VEA,max=4.5V | 17.0 mΩ |
| AND9925 eq. 11, VEA,max=5.0V | 19.2 mΩ |
| LS/LPT minimum practical RCS | ≥11.5 mΩ |

Therefore:

RCS=15mΩ is the best practical starting value

Why 15 mΩ?

- matches AN4165 example style

- keeps RLSinside range

- gives good current-sense signal

- reduces noise sensitivity versus 7 mΩ

- still keeps shunt loss manageable

- aligns with AND9925 if VEA,max≈4.0–4.1V

If you want the control window closer to VEA,max=4.5V, use:

RCS=16.5mΩ to 17mΩ

But my practical recommendation remains:

15mΩ, 1%, Kelvin-sensed, perphase

1. Key design values used

From your 2-channel interleaved design document:

VOUT=390V
fSW=70kHz
Lϕ=235μH
COUT=2200μF
NCH=2
POUT,LL=1700W
POUT,HL=3600W

The design document shows the 90 Vac low-line case, Vin,pk=127.279V, Dpk=0.673643, Iin,pk=28.304A, and selected Lϕ=235μH.

For FAN9672 universal input:

RIAC=6MΩ

For high-line-only setting:

RIAC=12MΩ

AN4165 gives this split between universal and high-voltage single-range operation.

2. RCS calculation from AN4165 equation 31

AN4165 selects the current-sense resistor from the maximum power limit per PFC stage:

RCS=VLINE,min2⋅GMAX⋅RMRIAC⋅POUT,max,phase

AN4165 also states that GMAX=2, RM=7.5kΩ, and that the maximum power limit is typically around 120–150% of full load.

Use:

KMAX=1.3

2.1 Low-line design

Low-line full power:

POUT,LL=1700W

Per phase:

POUT,LL,ϕ=17002=850W

Maximum phase power:

POUT,max,LL,ϕ=1.3×850=1105W

Use universal-input setting:

VLINE,min=90V
RIAC=6MΩ
RCS,LL=902⋅2⋅75006×106⋅1105
RCS,LL=18.3mΩ

So AN4165 low-line method suggests:

RCS≈18mΩ

2.2 High-line design

High-line full power:

POUT,HL=3600W

Per phase:

POUT,HL,ϕ=36002=1800W
Maximum phase power:

POUT,max,HL,ϕ=1.3×1800=2340W

For high-line-only operation:

VLINE,min=180V
RIAC=12MΩ
RCS,HL=1802⋅2⋅750012×106⋅2340
RCS,HL=17.3mΩ

So AN4165 high-line method suggests:

RCS≈17mΩ

2.3 Why 15mΩ is still acceptable

Your chosen:

RCS=15mΩ

is slightly below the 17–18mΩ calculated values, so it gives a slightly higher allowed power window before VEAsaturates. This is acceptable as a practical value if current-limit, LS/LPT, and loop-compensation checks pass.

3. RCS calculation from AND9925 equation 11

AND9925 gives the more general gain-modulator-based current-sense equation:

RCS=KRMRIAC(VEA,max-0.6)8KRLPK2RRLPK2(PMAX/NCH)

AND9925 explains that the FAN967x gain modulator creates the current command from IAC, peak line information, and VEA, and it also notes that the simplified AN4165 loop equation is reasonable for design, while fuller equations are available when RCSdiffers from the nominal equation value.

Use:

KRM=6000
KRLPK=2.465
RRLPK=12.4kΩ

3.1 Low-line implied VEA,maxwith RCS=15mΩ

Rearrange AND9925 equation:

VEA,max=0.6+RCS⋅8KRLPK2RRLPK2(PMAX/NCH)KRMRIAC

Low line:

PMAX/NCH=1105W
RIAC=6MΩ
VEA,max=0.6+0.015⋅8⋅2.4652⋅(12.4k)2⋅11056000⋅6M
VEA,max,LL=4.04V

This is good because AN4165 says the maximum power limit is commonly set so VEAis around 4–4.5V at maximum-power condition.

3.2 High-line implied VEA,maxwith RCS=15mΩ

High-line-only setting:

PMAX/NCH=2340W
RIAC=12MΩ
VEA,max=0.6+0.015⋅8⋅2.4652⋅(12.4k)2⋅23406000⋅12M
VEA,max,HL=4.24V

This is also in the preferred 4–4.5V control window.

3.3 RCS from AND9925 if targeting exact VEA,max

For reference:

| Case | VEA,max=4.0V | VEA,max=4.5V | VEA,max=5.0V |
| --- | --- | --- | --- |
| Low-line, RIAC=6MΩ, 1105W/phase | 14.8mΩ | 17.0mΩ | 19.2mΩ |
| High-line-only, RIAC=12MΩ, 2340W/phase | 14.0mΩ | 16.1mΩ | 18.1mΩ |

So:

RCS=15mΩ

maps to about 4.04V low-line and 4.24V high-line, which is a very reasonable compromise.

4. Operating-point calculations: low line and high line

4.1 Low-line, 90Vac, 1700W

From the design document:

Vin,pk=127.279V
D=1-127.279390=0.6736
Iin,rms=20.014A
Iin,pk=28.304A

Per-phase average crest current:

Iϕ,avg,crest=Iin,pk2
Iϕ,avg,crest=14.152A

Per-phase ripple at crest from your design:

ΔIL,ϕ,pp=5.212A

Per-phase peak current:

Iϕ,pk=Iϕ,avg,crest+ΔIL,ϕ,pp2
Iϕ,pk=14.152+5.2122
Iϕ,pk,LL=16.76A

Current-sense voltage at crest peak:

VCS,pk,LL=Iϕ,pk,LLRCS
VCS,pk,LL=16.76×0.015
VCS,pk,LL=251mV

Per-phase RMS from your report:

Iϕ,rms,LL=10.07A

Shunt loss:

PRCS,LL=Iϕ,rms2RCS
PRCS,LL=10.072×0.015
PRCS,LL=1.52W per phase

4.2 High-line, 180Vac, 3600W

From the design document:

Vin,pk=254.558V
D=1-254.558390=0.3473
Iin,rms=20.958A
Iin,pk=29.639A

Per-phase average crest current:

Iϕ,avg,crest=29.6392=14.820A

Per-phase ripple at crest:

ΔIL,ϕ,pp=5.374A

Per-phase peak current:

Iϕ,pk=14.820+5.3742
Iϕ,pk,HL=17.51A

Current-sense voltage:

VCS,pk,HL=17.51×0.015
VCS,pk,HL=263mV

Per-phase RMS from your report:

Iϕ,rms,HL=10.58A

Shunt loss:

PRCS,HL=10.582×0.015
PRCS,HL=1.68W per phase

| Item | Final value |
| --- | --- |
| Controller | FAN9672 analog average-current-mode PFC |
| Phases | 2 interleaved channels |
| VOUT | 390 V |
| fSW | 70 kHz |
| Lϕ | 235 µH per phase |
| COUT | 2200 µF |
| RCS | 15 mΩ per phase |
| Current loop crossover | ~8 kHz |
| Current loop zero | ~1.02 kHz |
| Current loop pole | ~26 kHz |
| Current-sense RC filter | 2 kΩ / 470 pF |
| Voltage-loop compensator | Type III, balanced option |
| Voltage-loop zeros | 3 Hz, 10 Hz |
| Voltage-loop poles | 17 Hz, 55 Hz |

Your design document gives the key plant values: VOUT=390V, fSW=70kHz, Lϕ=235µH, low-line Vin,pk=127.279V, low-line duty D=0.6736, and selected per-phase inductor ripple 5.212App.

1. FAN9672 control structure

FAN9672 has two nested loops:

Inner loop: average current loop

This loop makes each phase inductor current follow the current command.

FAN9672 equation:

ILRCS=IMORM⋅Gain2

For channel 1:

Gain2=1

For channel 2 at full load:

Gain2≈1

FAN9672’s datasheet describes the current-control loop as regulating the sensed inductor-current voltage ILRCSto the gain-modulator current command.

Outer loop: voltage loop

The voltage loop regulates the PFC bus by forcing:

VFBPFC=2.5V

The voltage error amplifier output is VEA. That VEAsignal feeds the gain modulator, which then generates the current command for the inner current loop.

So the control path is:

VOUT→Hv→GMV→VEA→Gain Modulator→GMI→PWM→Inductor Current→Output Bus

2. Voltage feedback divider

FAN9672 regulates the feedback pin to:

VFBPFC=2.5V

Therefore the divider gain is:

Hv=VFBPFCVOUT
Hv=2.5390
Hv=0.00641

This means a 1 V change on the PFC bus appears as only:

1V×0.00641=6.41mV

at the FAN9672 feedback pin.

3. Duty-cycle operating point

For boost-type CCM PFC:

D=1-VINVOUT

At low-line crest:

VIN,pk=127.279V
DLL=1-127.279390
DLL=0.6736

At 180 Vac high-line crest:

VIN,pk=254.558V
DHL=1-254.558390
DHL=0.3473

Your design report uses the same duty expression in the input/duty calculation table.

4. Per-phase power-stage current plant

For one boost phase in CCM:

LϕdiLdt=vin-(1-d)vo

Small-signal form:

LϕsiL(s)=VOUTd(s)

for current-loop design, where bus voltage and line-voltage perturbations are treated as slow relative to the current loop.

Therefore:

Gid(s)=iL(s)d(s)=VOUTLϕs
Gid(s)=390235μH⋅s
Gid(s)=1.66×106s

AND9925 shows the same FAN967x simplification:

Gid(s)=VOUTsL

when the current-loop bandwidth is above the low-frequency LC region.

5. PWM modulator gain

FAN9672 uses a voltage-mode PWM comparator. The PWM gain is:

Fm=1VRAMP

For FAN967x:

VRAMP≈5V

So:

Fm=0.2

AND9925 defines the PWM modulation gain as the reciprocal of the ramp amplitude.

6. Current-sense gain

You selected:

RCS=15mΩ=0.015Ω

So the sensed voltage is:

vCS=iLRCS

At low-line crest, from your design:

Iϕ,avg=14.152A
ΔIL,pp=5.212A

Peak phase current:

Iϕ,pk=14.152+5.2122
Iϕ,pk=16.758A

Sense voltage:

VCS,pk=16.758×0.015
VCS,pk=251mV

At high-line 180 Vac crest:

Iin,pk=29.639A
Iϕ,avg=14.819A
ΔIL,pp=5.374A
Iϕ,pk=14.819+5.3742
Iϕ,pk=17.506A
VCS,pk=17.506×0.015
VCS,pk=263mV

7. Current-loop open-loop equation

The FAN967x current-loop gain is:

TI(s)=Gid(s)⋅RCS⋅Gmi(s)⋅Fm

AND9925 gives this same block form:

TI(s)=Gid(s)RCSGmi(s)Fm

where Gmi(s)is the GMI current amplifier plus the IEA compensation network.

AN4165 gives the duty-to-inductor-current plant:

Gids=iLd=VPFCsLPFC

and the current-sense-voltage plant:

vCSvIEA=RCSVPFCVRAMPsLPFC

Substitute the plant:

TI(s)=VPFCLϕs⋅RCS⋅Gmi(s)⋅1VRAMP
TI(s)=390235μH⋅s⋅0.015⋅0.2⋅Gmi(s)
TI(s)=4979s⋅Gmi(s)

fCI=8kHz
fZ=1kHz
fP=26kHz
RF=2kΩ,CF=470pF

The current-sense RC filter pole is:

fRC=12πRFCF
fRC=12π⋅2000⋅470pF
fRC=169.3kHz

FAN9672 uses average-current-mode control with GMI transconductance current amplifiers, and the IEA pin compensation network sets the current-loop response. The FAN967x block diagram also shows the current-regulator path using CS pins, GMI, IEA, PWM, and OPFC output.

1. Current-loop plant at 8 kHz

Using the FAN967x / AN4165 current-loop plant form:

∣vCSvIEA∣=RCSVPFCVRAMP⋅2πfCILPFC

Use:

RCS=0.015Ω
VPFC=390V
VRAMP=5V
LPFC=235μH
fCI=8kHz
∣vCSvIEA∣=0.015⋅3905⋅2π⋅8000⋅235μH
∣vCSvIEA∣8kHz=0.099
This current-loop plant gain is the same for low-line and high-line in the simplified AN4165/FAN967x method because the plant uses VPFC, LPFC, RCS, VRAMP, and fCI, not line voltage.

So the compensator must provide approximately:

10.099=10.1

at 8 kHz.

8. Current-sense RC filter

You selected:

RF=2kΩ
CF=470pF

The filter pole is:

fRC=12πRFCF
fRC=12π⋅2000⋅470pF
fRC=169.3kHz

At current crossover:

fCI=8kHz

Filter magnitude:

HRC​(s)=1/(1+s/ωRC​1)

At 8 kHz: ​

∣HRC∣=11+(fCI/fRC)2
∣HRC∣=11+(8000/169300)2
∣HRC∣=0.999

Filter phase:

ϕRC=-tan⁡-18000169300
ϕRC=-2.7∘

So the filter removes switching noise but does not materially reduce current-loop phase margin.

5. Required compensator gain at 8 kHz

Including the RC filter:

∣GC(8kHz)∣=10.099⋅0.999
∣GC(8kHz)∣=10.1

For the compensator:

∣GC(fCI)∣=fIIfCI⋅1+(fCI/fZ)21+(fCI/fP)2

Solving gives:

fII=10.5kHz

6. Calculate IEA compensation components

Using FAN9672 GMI:

GMI=88μS

The integrator frequency is:

fII=GMI2πCIC1

So:

CIC1=GMI2πfII
CIC1=88μS2π⋅10.5k
CIC1=1.33nF

Now set the zero:

fZ=12πRICCIC1
RIC=12πfZCIC1
RIC=12π⋅1k⋅1.33nF
RIC=119kΩ

Now set the high-frequency pole:

fP=12πRICCIC2
CIC2=12πRICfP
CIC2=12π⋅119k⋅26k
CIC2=51pF

Practical values:

CIC1=1.3nF
RIC=118kΩ or 120kΩ
CIC2=51pF or 56pF

7. Gain margin estimate

9. Current-loop compensator form

The average-current-loop compensation uses the FAN9672 GMI transconductance amplifier and external IEA components.

The compensation form is:

Gmi(s)=2πfIIs⋅1+s/(2πfZ)1+s/(2πfP)

Your finalized values:

RIC=120kΩ
CIC1=1.3nF
CIC2=51pF

10. Actual current-loop zero

fZ=12πRICCIC1
fZ=12π⋅120k⋅1.3nF
fZ=1.02kHz

This zero adds phase boost before crossover.

11. Actual current-loop pole

fP=12πRICCIC2
fP=12π⋅120k⋅51pF
fP=26.0kHz

This pole limits high-frequency noise gain.

12. Current-loop crossover and margins

At approximately:

fCI≈8.1kHz

phase contributions are:

Power plant:

ϕplant=-90∘

Compensator:

ϕcomp=-90∘+tan⁡-1fCIfZ-tan⁡-1fCIfP
ϕcomp≈-24∘

Current-sense filter:

ϕRC≈-2.7∘

Total phase:

ϕtotal=-90-24-2.7
ϕtotal≈-116.7∘

Phase margin:

PM=180∘+ϕtotal
PM≈63∘

Gain margin:

The loop reaches -180∘near:

f180≈64kHz

The loop magnitude there is about:

∣T(jf180)∣≈0.045
GM=-20log⁡10(0.045)
GM≈27dB

This is a strong current-loop result.

13. Why current loop must be much faster than voltage loop

Your current loop crosses near:

8kHz

The voltage loop crosses in the low-Hz range.

So:

fCIfCV≫1

This separation is intentional. It lets the voltage loop command a current amplitude while the current loop quickly forces the actual inductor current to follow that command.

AND9925 states that the voltage-regulation bandwidth is generally lower in PFC converters, and within that bandwidth the current-tracking loop can be treated as much larger than one.

14. Voltage-loop plant

The PFC voltage loop regulates output-bus energy.

Capacitor energy:

E=12COUTVOUT2

Power balance:

COUTVOUTdvOUTdt=pin-pout

AN4165 gives the simplified low-frequency voltage plant for FAN967x line-feedforward PFC as:

vPFCvVEA≈IOUT,TOTKMAX5sCOUT

where 5Vis the voltage amplifier linear window.

At full power:

POUT=3600W
IOUT,TOT=POUTVOUT
IOUT,TOT=3600390
IOUT,TOT=9.23A

Using the finalized design basis:

KMAX=1.3
COUT=2200μF
Gvo(s)=9.23⋅1.35⋅2200μF⋅s
Gvo(s)=1090s

15. Why Type III voltage compensation was selected

The standard FAN9672 / AN4165 method uses a Type-II voltage compensator. But Type II gives limited phase boost and can become underdamped when the voltage-loop crossover is pushed higher.

TI’s OTA compensator note explains that OTA Type II uses an origin pole, one zero, and one high-frequency pole, while OTA Type III adds another pole-zero pair. It also warns that OTA Type III has less flexibility than op-amp Type III because the feedback divider participates in the AC response.

You selected Type III because it gives a better balance of:

- phase margin,

- load-step damping,

- 100/120 Hz rejection,

- THD risk,

- transient response.

16. Final voltage-loop Type III shape

Final selected voltage-loop shape:

Gmv(s)=Ks⋅1+s/ωZ1)(1+s/ωZ21+s/ωP1)(1+s/ωP2

with:

fZ1=3Hz
fZ2=10Hz
fP1=17Hz
fP2=55Hz

So:

Gmv(s)=Ks⋅1+s/2π3)(1+s/2π101+s/2π17)(1+s/2π55

This is a 3-pole / 2-zero loop shape if the origin pole is counted.

17. Voltage-loop phase explanation

At the selected crossover, each pole and zero contributes phase.

The origin pole contributes:

-90∘

Zero at 3 Hz contributes:

+tan⁡-1f3

Zero at 10 Hz contributes:

+tan⁡-1f10

Pole at 17 Hz contributes:

-tan⁡-1f17

Pole at 55 Hz contributes:

-tan⁡-1f55

At f=17Hz:

ϕcomp=-90+tan⁡-1(17/3)+tan⁡-1(17/10)-tan⁡-1(17/17)-tan⁡-1(17/55)
ϕcomp≈-12.6∘

This is much better than a Type-II voltage compensator at the same crossover.

18. Voltage-loop 100 / 120 Hz rejection

At 100–120 Hz, we do not want the voltage loop to strongly respond to twice-line-frequency bus ripple.

Reason:

VEA ripple→current command ripple→input current distortion→THD

The Type III selected option gives:

∣TV(100Hz)∣≈-19.8dB
∣TV(120Hz)∣≈-22.6dB

at the 17 Hz normalized crossover case.

That gives useful 100/120 Hz attenuation while preserving much better damping than Type II.

19. Step-load behavior

For 10% to 100% load step:

PSTEP=3600-360=3240W

The output capacitor initially supplies the missing energy until the voltage loop raises VEAand increases the current command.

For 100% to 10% load step, the opposite occurs: the PFC initially delivers too much energy, causing bus overshoot until VEAreduces the current command.

Estimated result with selected Type III at 17 Hz:

ΔVdip≈31V
ΔVovershoot≈31V

This was chosen because it gives better damping than Type II, even if Type II may show slightly smaller first dip in a simplified estimate.

20. Final loop summary

Current loop

RCS=15mΩ
RIC=120kΩ
CIC1=1.3nF
CIC2=51pF
fZ=1.02kHz
fP=26.0kHz
fCI≈8.1kHz
PM≈63∘,GM≈27dB

Current-sense filter

RF=2kΩ,CF=470pF
fRC=169.3kHz

Voltage loop

Type III OTA compensation
fZ1=3Hz,fZ2=10Hz
fP1=17Hz,fP2=55Hz
PM≈77∘
100Hz loop gain≈-19.8dB
120Hz loop gain≈-22.6dB

Final engineering conclusion

Your finalized loop design is well balanced.

The current loop is fast and stable:

fCI≈8.1kHz,PM≈63∘,GM≈27dB

The current-sense RC filter is high enough not to damage phase margin:

fRC=169kHz

The voltage loop uses Type III shaping to improve damping while still attenuating 100/120 Hz ripple:

z1=3Hz, z2=10Hz, p1=17Hz, p2=55Hz

So the recommended final analog feedback-loop architecture is:

FAN9672 average current loop:RCS=15mΩ, RIC=120k, CIC1=1.3nF, CIC2=51pF
Voltage loop:Type III OTA, z1=3Hz, z2=10Hz, p1=17Hz, p2=55Hz

This gives the best practical balance of current tracking, voltage-loop damping, line-frequency rejection, THD risk, and load-step recovery.

8. LS / LPT and GC recalculation

AN4165 uses LS and GC to implement the linear-predict current emulator. FAN9672 also identifies LS as the current-predict function setting and GC as the input-voltage gain-control pin.

Use:

VOUTVFB=3902.5=156
RLS=LPFC1.5×10-9⋅RCS⋅(VOUT/VFB)
RLS=235μH1.5×10-9⋅0.015⋅156
RLS=66.95kΩ

Use:

RLS=66.5kΩ or 68kΩ

GC:

RGC=6MΩVOUT/VFB
RGC=6M156
RGC=38.46kΩ

Use:

RGC=38.3kΩ or 39kΩ

9. Oscillator resistor

FAN9672 frequency is set by RI:

fOSC=8×108RRI

and FAN9672 allows 55–75kHz operation in the high-frequency range.

RRI=8×10870k
RRI=11.43kΩ

Use:

RRI=11.3kΩ or 11.5kΩ

10. Voltage-loop plant and compensation

AN4165 gives the low-frequency control-to-output plant for line-feedforward PFC as:

vPFCvVEA≈IOUT,TOTKMAX5⋅sCOUT

and then the voltage compensation equations:

CVC1=GMV⋅IOUT,TOT⋅KMAX5COUT(2πfVC)2⋅2.5VPFC
RVC=12πfVCCVC1
CVC2=12πfVPRVC

where fVPshould be at least one decade above fVC.

Use:

GMV=100μS
KMAX=1.3
fVC=17Hz
fVP=10fVC=170Hz