Step 1) Power Plant Specs:

- VOUT=393.7 V

- fSW=70 kHz

- Lϕ=235 μH

- COUT=2200 μF

- POUT,LL=1700 W

- POUT,HL=3600 W

- NCH=2

These are treated as already completed power-stage values. [FAN9672 Architecture | Word]

Step 2) Set the Base

- Iac maximum value less than 65uA

- Vlpk maximum value should be less than 3.8Vpreferble to be less or equal to 3.7V

- Krlpk = 2.465

- Krm = 6000

- Vea,max = 5.6. Desirable window is 4V~5V.

- Rlpk = 12.1K

- Rvir = 10 KOHM for low range

- Rvir = 470 KOHM for high Range

- Kmax value should be between 1.3 to 1.6. Select the value so that Rcs can satisfy both equations considered during calculation.

Step 3) Gain Modulator and IAC Check

VIN Range and RIAC Setting

For this design,

Lower input AC voltage range 90VAC to 132VAC

Iac = Vac*1.414/Rac

= 132*1.414/6M = 31.11uA

Higher input AC voltage range 180VAC to 264VAC

Iac = Vac*1.414/Rac

= 264*1.414/12M = 31.11uA

VAC=85 V to 264 V

RIAC=6 MΩ or 12MΩ

31.11 μA<65 μA

- Use 1% or better resistor tolerance and split RIAC into multiple high-voltage resistors for voltage rating and creepage.

Step 4)  Oscillator Resistor Check for Selected FSW

The FAN9672-D gives the oscillator equation:

fOSC=8×108RRI

The same equation is used in AN4165-D. The FAN9672 supports the high-frequency range around 55 kHz to 75 kHz, so the selected 70 kHz is valid.

For:

fSW=70 kHz

RRI=8×10870×103

RRI=11.43 kΩ

Recommended value:

RRI=11.3 kΩ or 11.5 kΩ

Decision:
Use 11.3 kΩ, 1% if you want slightly higher frequency, or 11.5 kΩ, 1% if you want slightly lower frequency.

Step 5) Output Sensing and PVO Setting

The FAN9672 regulates the FBPFC pin to:

VFBPFC=2.5 V

The FAN9672-D and AN4165-D show the output-voltage divider and PVO function. The PVO pin changes the effective feedback target using:

VFBPFC=2.5-VPVO4

The PVO function is disabled when PVO is below the disabled threshold, and the datasheet example uses 0 V for normal high-output operation.

Use the standard lower divider resistor:

RFB4=23.2 kΩ

For target:

VOUT=393.7 V

Total divider resistance:

RTOTAL=RFB4VOUTVFBPFC

RTOTAL=23.2kΩ×393.72.5

RTOTAL=3.6532 MΩ

Upper divider resistance:

RFB1+RFB2+ +RFB3=3.697 MΩ-23.2 kΩ

RFB1+RFB2+ +RFB3=3.63 MΩ

Practical selection:

RFB1=1.21 MΩ

RFB2=1.21 MΩ

RFB3=1.21 MΩ

Actual output:

VOUT=2.5⋅1.21M+1.21M+1.21M+23.2k23.2k

VOUT=393.7 V

Important PVO warning:
AN4165-D and FAN9672-D state that the PFC output voltage should be kept at least 25 V higher than the AC input peak voltage when using PVO. At 264 Vac:

VIN,pk=373.35 V

With 390 V bus:

393.7-373.35=20.35 V

So the 390 V bus does not provide 25 V margin at 264 Vac.

Decision:

- For this design, keep PVO = 0 V / disabled during high-line operation.

- Do not reduce the bus to 350 V at high line.

- If PVO is used, only enable it where the reduced bus still remains at least 25 V above VIN,pk.

Step 6) RCS Calculation Two Ways

AN4165 Equation 31 and AND9925 Equation 11

Your existing FAN9672 Architecture already uses:

- VOUT=393.7 V

- fSW=70 kHz

- Lϕ=235 μH

- COUT=2200 μF

- POUT,LL=1700 W

- POUT,HL=3600 W

- NCH=2

6.1 AN4165 Equation 31 Method

AN4165-D gives:

RCS=VLINE,min2⋅GMAX⋅RMRIAC⋅POUT,max,phase

where AN4165 uses:

GMAX=2

RM=7.5 kΩ

and recommends setting maximum power around 120% to 150% of full load. [AN4165-D | PDF]

Use 149% margin:

KMAX=1.49

Low-line per-phase full power:

Pphase,LL=17002=850 W

Pmax,phase,LL=1.49×850=1266.5 W
At 90 Vac:

RCS=902⋅2⋅75006MΩ⋅1266.5

RCS=15.98 mΩ

For high-line-only reference using 180 Vac, 3600 W, and RIAC = 12 MΩ:

Pphase,HL=36002=1800 W

Pmax,phase,HL=1.49×1800=2682 W

RCS=1802⋅2⋅750012MΩ⋅2682

RCS=15.1 mΩ
AN4165 result range:

RCS≈15 mΩ

6.2 AND9925 Equation 11 Method

AND9925-D gives the general gain-modulator-based current-sense equation:

RCS=KRMRIAC(VVEA,max-0.6)8KRLPK2RRLPK2(PMAX/NCH)

AND9925 states that:

KRM≈6000

KRLPK=2.465

RRLPK=12.1 kΩ

VVEA,max=5.1V

and that VVEA,maxis generally set around 4 V to 5 V. [AND9925-D | PDF]

For low line:

PMAX/NCH=1266.5 W

RIAC=6.0MΩ

| Target VEA,max | RCS from AND9925 |
| --- | --- |
| 4.0 V | 13.56 mΩ |
| 4.5 V | 15.57 mΩ |
| 5.0 V | 17.57 mΩ |
| 5.1 V | 17.97 mΩ |

For high-line-only reference:

PMAXNCH=2682 W
RIAC=12MΩ

| Target VEA,max | RCS from AND9925 |
| --- | --- |
| 4.0 V | 12.825 mΩ |
| 4.5 V | 14.711 mΩ |
| 5.1 V | 16.97 mΩ |

6.3 Verification of Selected RCS = 15 mΩ

For the selected:

RCS=15 mΩ

The implied low-line VEA,max is:

VVEA,max,LL=4.04 V

The implied high-line VEA,max is:

VVEA,max,HL=4.24 V

This sits inside the preferred 4 V to 5 V operating window discussed in AND9925-D and aligns with the AN4165 practical maximum-power setting approach.

Decision:

RCS=15 mΩ, 1%, Kelvin sensed, per phase
Reason:

- AN4165 gives about 16 mΩ to 18 mΩ.

- AND9925 gives about 15 mΩ to 19 mΩ for VEA,max from 4 V to 5 V.

- 15 mΩ keeps VEA,max near 4.0 V to 4.25 V, which is a stable control-window choice.

- 15 mΩ also keeps the LS resistor inside the FAN9672 allowed range.

6.4  Power Drop Calculation for Selected RCS

Use:

RCS=15 mΩ

From FAN9672 Architecture, the already calculated per-phase RMS currents are approximately:

Iϕ,rms,LL=10.07 A

Iϕ,rms,HL=10.58 A

and the phase peak currents are approximately:

Iϕ,pk,LL=16.76 A

Iϕ,pk,HL=17.51 A

Low Line

VCS,pk,LL=Iϕ,pk,LLRCS

VCS,pk,LL=16.76×0.015

VCS,pk,LL=251 mV

Power loss per phase:

PRCS,LL=Iϕ,rms,LL2RCS

PRCS,LL=10.072×0.015

PRCS,LL=1.52 W per phase

Two phases:

PRCS,total,LL=3.04 W

High Line

VCS,pk,HL=17.51×0.015

VCS,pk,HL=263 mV

Power loss per phase:

PRCS,HL=10.582×0.015

PRCS,HL=1.68 W per phase

Two phases:

PRCS,total,HL=3.36 W

Decision:
Use at least 3 W rated shunt per phase, preferably with thermal margin. A 4-terminal/Kelvin metal-element shunt is strongly recommended.