the outer voltage loop is harder than the current loop because it does not directly control duty cycle. In FAN9672, the voltage loop controls the amplitude of the sinusoidal current reference through the multiplier/gain-modulator block.

Think of it this way:

VOUT→Voltage Divider→GMV Amplifier→VEA→Multiplier→Current Command→Current Loop→PFC Power Stage→VOUT

The current loop is “fast current tracking.”
The voltage loop is “slow energy balancing.”

1. What the outer voltage loop is trying to do

The PFC output capacitor stores energy:

E=12COUTVOUT2

If load power suddenly increases, the capacitor supplies the missing energy and VOUTdips.

If load power suddenly decreases, extra input power charges the capacitor and VOUTovershoots.

The voltage loop watches VOUT, compares it to the 390 V target, and adjusts VEA. FAN9672 then uses VEAinside the multiplier to increase or decrease the input current command.

So the voltage loop does not say:

change duty directly

It says:

increase or decrease input-current amplitude

2. Voltage-loop plant derivation from energy balance

Start with output capacitor power balance:

PCAP=PIN-POUT

Capacitor energy is:

E=12COUTVOUT2

Differentiate energy:

dEdt=COUTVOUTdVOUTdt

Therefore:

COUTVOUTdVOUTdt=PIN-POUT

This is the most important voltage-loop equation.

For small-signal control, assume POUTis a disturbance and the loop controls PIN. Around the operating point:

COUTVOUTsvOUT(s)=pIN(s)

So:

vOUT(s)pIN(s)=1COUTVOUTs

This means the bus voltage plant is an integrator:

Gvp(s)=vOUT(s)pIN(s)=1COUTVOUTs

This is why the voltage loop is slow.

3. FAN9672 version of the voltage plant

ON Semiconductor simplifies the FAN967x voltage plant as:

vPFCvVEA≈IOUT,TOTKMAX5sCOUT

This form says:

VEA→input current amplitude→input power→output capacitor→VOUT

The “5” comes from the approximate useful linear control range of the VEA / multiplier path in FAN967x-style design. Your corrected report also uses this low-frequency voltage-loop framework.

For your design:

POUT=3600W
VOUT=390V
IOUT,TOT=POUTVOUT=3600390=9.23A
COUT=2200μF
KMAX=1.3

So:

Gvo(s)=9.23×1.35×2200μF×s
Gvo(s)=11.9990.011s
Gvo(s)≈1090s

This is the voltage-loop plant from VEAto VOUT.

4. What are the voltage-loop blocks?

The outer voltage loop has these blocks:

TV(s)=Hv⋅Gmv(s)⋅GMOD(s)⋅Gpower(s)

For FAN9672, conceptually:

| Block | Meaning |
| --- | --- |
| Hv | Output-voltage divider gain |
| Gmv(s) | Voltage error amplifier + compensation network |
| GMOD | Multiplier / gain-modulator block |
| Current loop | Usually treated as unity because it is much faster |
| Gpower(s) | Output capacitor energy plant |

Because current loop crossover is about:

fCI≈8.1kHz

and voltage loop is:

fCV=17Hz

we can say:

Gcurrent,closed(s)≈1

inside the voltage-loop bandwidth.

5. Output-voltage divider gain

FAN9672 regulates:

VFBPFC=2.5V

Your bus is:

VOUT=390V

So the divider gain is:

Hv=2.5390
Hv=0.00641

This means the controller only sees a scaled version of the output bus.

A 1 V bus change becomes:

1V×0.00641=6.41mV

at the feedback pin.

6. What does the multiplier do?

The multiplier converts voltage-loop output VEAinto a sinusoidal current command.

Conceptually:

ICMD∝IAC⋅VEAVLPK2

where:

| Term | Meaning |
| --- | --- |
| IAC | Instantaneous rectified line-voltage signal |
| VEA | Output of voltage loop |
| VLPK | Line peak feed-forward signal |
| VLPK2 | Feed-forward normalization |

So:

VEA↑⇒ICMD↑⇒PIN↑⇒VOUT↑

and:

VEA↓⇒ICMD↓⇒PIN↓⇒VOUT↓

That is the outer loop action.

7. Why the voltage loop must be slow

The PFC output naturally has double-line-frequency ripple:

fripple=2fline

So:

50Hz line⇒100Hz ripple
60Hz line⇒120Hz ripple

If the voltage loop has high gain at 100/120 Hz, then VEAwill contain 100/120 Hz ripple.

That causes:

VEA ripple→current command ripple→input current distortion→higher THD

So we intentionally keep:

fCV≪100/120Hz

Your selected:

fCV=17Hz

is aggressive but still below 100/120 Hz.

8. Why Type III was selected for voltage loop

A Type II OTA compensator has:

pole at origin+1 zero+1 high-frequency pole

A Type III OTA compensator adds another pole-zero pair. TI SLVA662 explains that OTA Type III adds another RC branch and gives additional phase boost, but with less flexibility than op-amp Type III because both feedback-divider resistors participate in AC behavior.

Your final selected voltage-loop Type III shape is:

Gmv(s)=Ks⋅1+s/ωZ1)(1+s/ωZ21+s/ωP1)(1+s/ωP2

with:

fZ1=3Hz
fZ2=10Hz
fP1=17Hz
fP2=55Hz

So:

Gmv(s)=Ks⋅1+s/2π3)(1+s/2π101+s/2π17)(1+s/2π55

9. How voltage-loop phase margin is calculated

At crossover:

fCV=17Hz

Each pole and zero contributes phase.

Origin pole

-90∘

Zero at 3 Hz

+tan⁡-1173=+80.0∘

Zero at 10 Hz

+tan⁡-11710=+59.5∘

Pole at 17 Hz

-tan⁡-11717=-45.0∘

Pole at 55 Hz

-tan⁡-11755=-17.2∘

Voltage plant phase

The corrected report uses:

-84.25∘

at 17 Hz.

So total phase:

ϕtotal=-90+80+59.5-45-17.2-84.25
ϕtotal=-96.9∘

Phase margin:

PM=180∘+ϕtotal
PM=180-96.9
PM≈83∘

That is very good.

10. Why Type II was not selected

For fair comparison, the corrected report compares Type II and Type III at the same:

fCV=17Hz

Type II used:

fZ=5Hz
fP=55Hz

Its phase is:

ϕT2=-90+tan⁡-1175-tan⁡-11755-84.25
ϕT2=-90+73.6-17.2-84.25
ϕT2=-117.8∘
PMT2=180-117.8
PMT2≈62∘

So Type III gives:

83∘-62∘=21∘

more phase margin.

That means better damping and more tolerance margin.

11. How the compensator gain Kis selected

The compensator gain is selected so the full voltage loop crosses 0 dB at:

fCV=17Hz

At crossover:

∣TV(jωCV)∣=1

So:

∣Hv∣⋅∣Gmv(jωCV)∣⋅∣GMOD∣⋅∣Gpower(jωCV)∣=1

Therefore:

K=1∣Hv∣⋅∣Gshape(jωCV)∣⋅∣GMOD∣⋅∣Gpower(jωCV)∣

Where:

Gshape(s)=1s⋅1+s/ωZ1)(1+s/ωZ21+s/ωP1)(1+s/ωP2

The exact Kdepends on the FAN9672 multiplier gain constants and OTA GMV. In practice, ON Semiconductor’s method folds the multiplier gain into the application equations for CVC1,RVC,CVC2.

So the design process is:

- Choose fCV

- Choose zeros/poles

- Compute required gain at fCV

- Convert required gain into VEA compensation impedance

- Select standard Rand C

- Verify actual crossover and phase margin

12. Beginner-friendly analogy

The outer voltage loop is like cruise control on a car.

The current loop is the engine throttle actuator: fast and precise.

The voltage loop is the driver watching speed: slower.

If the bus voltage dips, the voltage loop says:

increase current amplitude

If the bus voltage rises, it says:

decrease current amplitude

But it must not react to every 100/120 Hz ripple bump, otherwise it makes the input current waveform ugly.

Final summary

The outer voltage loop is closed like this:

VOUT→Hv→GMV+ZVEA(s)→VEA→Multiplier→Current Command→Fast Current Loop→Input Power→COUT→VOUT

The plant is derived from capacitor energy:

COUTVOUTdVOUTdt=PIN-POUT

Pout = Vout2/R

Linearized:

CoVosvo(s)=pin(s)-2VoRvo(s)

Therefore:

Gvp(s)=RVo(RCos+2)

For high-line full power:

R=39023600=42.25Ω
Co=2200μF
RCo=42.25×2200μF=0.09295
Gvp,HL(s)=42.25390(0.09295s+2)
Gvp,HL(s)=0.10830.09295s+2

For low-line full power:

R=39021700=89.47Ω
RCo=89.47×2200μF=0.1967
Gvp,LL(s)=89.47390(0.1967s+2)
Gvp,LL(s)=0.22940.1967s+2

The simplified FAN9672 voltage plant is:

vPFCvVEA≈IOUT,TOTKMAX5sCOUT

The selected Type III voltage-loop shape is:

z1=3Hz,z2=10Hz,p1=17Hz,p2=55Hz

and gives:

PM≈83∘

compared with Type II:

PM≈62∘

That is why Type III was selected: not because it is always simpler, but because it gives better damping, more phase margin, and a more robust 17 Hz voltage-loop design.