Step 1) Full Gid form for inner current loop use below equation:

Hcs(s) = Filter across current sense resistor

Gmi(s) = Current compensator = gmi * Ziea = Type 2 OTA

Rcs =Current sense Resistor

Means equation becomes:

Current Loop Gain Ti(s)

Where:

Now once we have inner current loop, next step is to find outer voltage loop.

Voltage Loop Gain Tv(s)

Where,

Now do step by step calculation using improved equations in attachment for

- Current Loop Gain Ti(s)

- Voltage Loop Gain  Tv(s)

Show gain margin and phase margin of each voltage and current loop along with step by step calculation‌ for

- Input voltage of 90VAC/110VAC/120VAC/132VAC for 1700W output and 180VAC/210VAC/220VAC/264VAC for 3600W output.

- Generate bode plot and summary table showing phase margin, gain margin and cross over at each voltage.

- Take Rcs = 15 mOHM, Vout = 393V, Nch = 2, Vramp = 5V, Co = 2200 uF, L = 235 uH per phase

- GMV = 100 u , GMI = 88 u , Kmax = 1.4, RL = 10 mOHM, rC = 10 mOHM, VFBPFC = 2.5, VRAMP = 5V

- Voltage feedback resistors: Vfb_top = 3.63 MOHM, Vfb_bottom = 23.2KOHM

- Inner current loop will be type 2 OTA with high frequency pole for current sensing at 169 KHz. (R =2KOHM and C = 470 pF).

- Current loop crossover frequency will be 8 KHz.

- Current loop will have Z1 at 1 KHZ and P1 at 26KHZ.

- Voltage loop cross over is at 17 Hz.

- Voltage loop compensator will be type 3 OTA.

- Voltage loop will have Z1 at 3Hz, Z2 at 12 Hz, P1 at 17 Hz and P2 at 50 Hz.

Generate bode plot showing Gain and phase margin for

- Open Inner Current Loop for all 8 voltages

- Closed inner current loop for all 8 voltages

- Open Voltage loop for low line voltages

- Open Voltage loop for High line voltages

- Closed voltage loop for low line voltages

- Closed voltage loop for High line voltages

Generate summary table having below columns showing

- VAC

- Pout

- Duty

- Fo (Hz)

- Q

- Current loop Fcrossover (KHz)

- Open Current loop Gain

- Open Current Loop Phase

- Voltage loop Fcrossover (KHz)

- Open Voltage loop Gain

- Open Voltage Loop Phase

- Closed Voltage loop Gain

- Closed Voltage Loop Phase

- 120 Hz Open loop Gain and Phase

- 120 Hz Closed loop Gain and Phase

Generate step load response and a summary table for all 8 voltages showing voltage dip or overshoot and recovery time for

- 0% to 100% step load

- 0% to 50% step load

- 50% to 100% step load

- 100% to 0% step load

- 50% to 0% step load

- 100% to 50% step load

We can not change power components which means inductor, capacitor etc. but only can modify voltage and current loop compensators' pole zero placement. Calculate and suggest how we can improve pole zero placement of both loops to improve transient response, better 120 Hz rejection and THD improvement.

Calculate current compensator components (R and C) values and Voltage compensator components (R & C) values. Suggest standard components.

Calculate other IC parameters

To calculate inner current loop components, Consider Ti(s) in below equation without compensator. Means:

RLOAD=VOUT2POUT

in the front gain factor and ωz.

1. Current loop without compensator

Ti(s)=VOUTLϕ⋅RLOAD+2rCRLOAD+rC⋅s+ωzs2+a1s+a0⋅RCSVRAMP⋅11+s/ωRC

This is without:

GMI⋅ZIEA(s)

2. Given values

VOUT=390V
Lϕ=235μH
CO=2200μF
rL=10mΩ
rC=10mΩ
RCS=15mΩ
VRAMP=5V
RF=2kΩ
CF=470pF
fci=8kHz

3. Example: 90 Vac, 1700 W

Load resistance

RLOAD=VOUT2POUT
RLOAD=39021700
RLOAD=89.4706Ω

Duty cycle

VIN,pk=2⋅90=127.279V
D=1-VIN,pkVOUT
D=1-127.279390
D=0.67364

4. Correct front factor

Kfront=VOUTLϕ⋅RLOAD+2rCRLOAD+rC
RLOAD+2rCRLOAD+rC=89.4706+2(0.01)89.4706+0.01
=1.00011
VOUTLϕ=390235μH
=1.6596×106
Kfront=1.6598×106

5. Calculate ωz

ωz=1CO(RLOAD/2+rC)
ωz=12200μF(89.4706/2+0.01)
ωz=10.1585 rad/s
fz=ωz2π
fz=1.617Hz

6. Calculate a1

a1=COrL(RLOAD+rC)+RLOADrC(1-D)2+LLCO(RLOAD+rC)

Substituting:

a1=2200μF[0.01(89.4706+0.01)+89.4706(0.01)(0.32636)2]+235μH235μH⋅2200μF(89.4706+0.01)
a1=52.1648

7. Calculate a0

a0=1-D)2RLOAD+rLLCO(RLOAD+rC)
a0=(0.32636)2(89.4706)+0.01235μH⋅2200μF(89.4706+0.01)
a0=2.06206×105

8. Evaluate at crossover

fci=8kHz
ω=2πf=2π(8000)
ω=50265.48 rad/s
s=j50265.48
Gid,14(s)=Kfront⋅s+ωzs2+a1s+a0

At 8kHz:

∣Gid,14(j2π8k)∣=33.0226
∠Gid,14=-89.952∘

9. Add RCS/VRAMP

RCSVRAMP=0.0155
=0.003
33.0226⋅0.003=0.0990677

10. Add current-sense filter

fRC=12πRFCF
fRC=12π(2000)(470pF)
fRC=169.314kHz
HCS(s)=11+s/ωRC

At 8 kHz:

∣HCS∣=11+(8000/169314)2
∣HCS∣=0.99889
∠HCS=-tan⁡-18000169314
∠HCS=-2.705∘

11. Final Ti(s)without compensator

∣Ti∣=∣Gid,14∣⋅RCSVRAMP⋅∣HCS∣
∣Ti∣=33.0226⋅0.003⋅0.99889
∣Ti(j2π8k)∣=0.09896

Gain in dB:

20log⁡10(0.09896)
-20.09dB

Phase:

∠Ti=∠Gid,14+∠HCS
∠Ti=-89.952∘-2.705∘
∠Ti=-92.657∘

Final answer at 90 Vac / 1700 W

∣Ti∣without compensator=0.09896
Ti=-20.09dB
∠Ti=-92.66∘

Across all operating points

| Vin | Pout | RLOAD| Duty D| ∣Ti∣at 8 kHz | Gain | Phase |
|---:|---:|---:|---:|---:|---:|---:|
| 90 Vac | 1700 W | 89.47 Ω | 0.6736 | 0.09896 | -20.09 dB | -92.66° |
| 110 Vac | 1700 W | 89.47 Ω | 0.6011 | 0.09896 | -20.09 dB | -92.65° |
| 120 Vac | 1700 W | 89.47 Ω | 0.5649 | 0.09896 | -20.09 dB | -92.65° |
| 132 Vac | 1700 W | 89.47 Ω | 0.5213 | 0.09897 | -20.09 dB | -92.65° |
| 180 Vac | 3600 W | 42.25 Ω | 0.3473 | 0.09899 | -20.09 dB | -92.65° |
| 210 Vac | 3600 W | 42.25 Ω | 0.2385 | 0.09901 | -20.09 dB | -92.64° |
| 220 Vac | 3600 W | 42.25 Ω | 0.2022 | 0.09901 | -20.09 dB | -92.64° |
| 264 Vac | 3600 W | 42.25 Ω | 0.0427 | 0.09903 | -20.08 dB | -92.62° |

So the correct uncompensated current-loop gain is:

∣Ti∣≈0.099

not 0.1487, because the front gain must use RLOAD, not rL.

Using:

∣Ti∣without compensator=0.09896
GMI=88μS
fci=8kHz
fz=1kHz
fp=26kHz

1. Current compensator form

Gmi(s)=GMI⋅ZIEA(s)
ZIEA(s)=RIC⋅1+ωz/s1+s/ωp

At crossover:

∣Ti(jωci)∣⋅GMI⋅∣ZIEA(jωci)∣=1

2. Compensator shape magnitude at 8 kHz

∣1+ωz/s1+s/ωp∣=1+(fz/fci)21+(fci/fp)2
=1+(1000/8000)21+(8000/26000)2
=0.9632

3. Calculate RIC

RIC=1∣Ti∣⋅GMI⋅0.9632
RIC=10.09896⋅88μS⋅0.9632
RIC=119.2kΩ

Recommended standard:

RIC=120kΩ

4. Calculate CIC1

fz=12πRICCIC1
CIC1=12πRICfz
CIC1=12π⋅119.2k⋅1k
CIC1=1.335nF

Recommended standard:

CIC1=1.3nF

5. Calculate CIC2

fp=12πRICCIC2
CIC2=12πRICfp
CIC2=12π⋅119.2k⋅26k
CIC2=51.35pF

Recommended standard:

CIC2=51pF

or:

CIC2=56pF

Final current compensator values

| Component | Calculated | Recommended standard |
| --- | --- | --- |
| RIC | 119.2 kΩ | 120 kΩ |
| CIC1 | 1.335 nF | 1.3 nF |
| CIC2 | 51.35 pF | 51 pF / 56 pF |

RIC=120kΩ,CIC1=1.3nF,CIC2=51pF

- Voltage loop compensator calculation:

Consider the equation below without compensator:

Below is the voltage-loop gain without the voltage compensator:

Tv(s)=KMAXIOUT5⋅VFBPFCVOUT⋅Ti(s)/NCH1+Ti(s)/NCH⋅1+sCOrC)(1-s/ωRHPCOs+2/RLOAD

This means I am not including:

GMV⋅Zcomp(s)
Method B means:

Hv=VFBPFCVOUT

is removed from Tv(s)and becomes part of the OTA Type-III compensator transfer function from VOto VEA, exactly as SLVA662 page 11–12 shows for OTA Type-III. In SLVA662, the OTA Type-III transfer function includes R1, R4, gm, and the compensation network together.

1. Voltage loop without feedback divider and without compensator

Use:

Tv,base(s)=KMAXIOUT5⋅Ti(s)/NCH1+Ti(s)/NCH⋅1+sCOrC)(1-s/ωRHPCOs+2/RLOAD

No:

VFBPFCVOUT

and no:

GMVZcomp(s)

1. Given values

KMAX=1.49
VOUT=390V
VFBPFC=2.5V
CO=2200μF
rC=10mΩ
NCH=2
fcv=17Hz
s=j2π17=j106.814

For the current-loop term, I used the corrected current compensator:

RIC=119.2kΩ
CIC1=1.335nF
CIC2=51.35pF

Example calculation: 180 Vac, 3600 W

2. Output current

IOUT=POUTVOUT

IOUT=3600390=9.2308A

3. Multiplier gain term

GMOD=KMAXIOUT5
GMOD=1.4⋅9.23085
GMOD=2.5846

4. Current-loop closed-loop term

Gi,cl(s)=Ti(s)/NCH1+Ti(s)/NCH

At 17 Hz, using the corrected current loop:

Ti(j2π17)=1619.85-j174.23
∣Ti∣=1629.19
∠Ti=-6.14∘

Now:

Gi,cl(j2π17)=Ti/21+Ti/2
Gi,cl=0.998781-j0.000131
∣Gi,cl∣=0.998781
∠Gi,cl=-0.0075∘

So at 17 Hz the current loop is almost unity, but it is still included.

6. Accurate voltage plant

Gvp(s)=1+sCOrC)(1-s/ωRHPCOs+2/RLOAD

Load resistance:

RLOAD=39023600
RLOAD=42.25Ω

Duty complement:

D'=2⋅180390
D'=0.6526

RHP zero:

ωRHP=RLOAD(D')2Leq
Leq=235μH2=117.5μH
ωRHP=153138 rad/s
fRHP=24.38 kHz

At 17 Hz:

Gvp(j2π17)=0.40553-j4.21692
∣Gvp∣=4.23637
∠Gvp=-84.51∘

7. Final voltage-loop gain without compensator

Tv=GMOD⋅Hv⋅Gi,cl⋅Gvp
Tv=1.22051⋅0.0064103⋅(0.998781-j0.000131)⋅(0.40553-j4.21692)
Tv=0.003165-j0.032953
∣Tv(j2π17)∣=0.03310
20log⁡10(0.03310)
Tv=-29.60dB
∠Tv=-84.51∘

Results across all operating points

| Vin | Pout | RLOAD| fRHP| ∣Tv∣at 17 Hz | Gain | Phase |
|---:|---:|---:|---:|---:|---:|---:|
| 90 Vac | 1700 W | 89.47 Ω | 12.91 kHz | 0.03310 | -29.60 dB | -84.51° |
| 110 Vac | 1700 W | 89.47 Ω | 19.28 kHz | 0.03308 | -29.61 dB | -84.49° |
| 120 Vac | 1700 W | 89.47 Ω | 22.95 kHz | 0.03307 | -29.61 dB | -84.49° |
| 132 Vac | 1700 W | 89.47 Ω | 27.77 kHz | 0.03305 | -29.62 dB | -84.48° |
| 180 Vac | 3600 W | 42.25 Ω | 24.38 kHz | 0.06878 | -23.25 dB | -78.57° |
| 210 Vac | 3600 W | 42.25 Ω | 33.19 kHz | 0.06865 | -23.27 dB | -78.58° |
| 220 Vac | 3600 W | 42.25 Ω | 36.42 kHz | 0.06861 | -23.27 dB | -78.58° |
| 264 Vac | 3600 W | 42.25 Ω | 52.45 kHz | 0.06838 | -23.30 dB | -78.61° |

Final answer

At the selected voltage-loop crossover:

fcv=17Hz

the voltage-loop gain without the voltage compensator is:

High Line 3600 W region

∣Tv,base∣=10.731
Tv,base=20.61dB
∠Tv,base=-78.57∘

So the required OTA Type-III compensator magnitude is:

∣HOTA(17Hz)∣=110.731
∣HOTA(17Hz)∣=0.09319
HOTA(17Hz)=-20.61dB
∠Tv≈-78.57∘

So the voltage compensator GMVZcomp(s)must provide approximately:

+29.6dB

at low-line / 1700 W and approximately:

+23.3dB

at high-line / 3600 W to force the loop to cross at 17 Hz.

NOw consider GMV = 100 u and find voltage loop compensator value for OTA 3.

Thought for 41s

Using:

GMV=100μS

and the previously calculated voltage-loop gain without compensator at fcv=17Hz:

Low-line 1700 W:

∣Tv∣no comp=0.03310

High-line 3600 W:

∣Tv∣no comp=0.06878

Voltage compensator must satisfy:

∣Tv∣no comp⋅∣GMVZcomp(jωcv)∣=1

2. SLVA662 OTA Type-III equations

SLVA662 defines the OTA Type-III normalized form as:

H(s)=-G01+ωz1/s)(1+s/ωz21+s/ωp1)(1+s/ωp2

with:

G0=R4gmR2C1R4+R1)(C1+C3

and the pole-zero definitions:

fz1=12πR2C1
fz2=12π(R1+R3)C2
fp1=C1+C32πR2C1C3
fp2=12πR4R1R4+R1R3C2

3. Given values

R1=RFB,TOP=3.63MΩ
R4=RFB,BOTTOM=23.2kΩ
gm=GMV=100μS
fc=17Hz
G=∣HOTA(17Hz)∣=0.093187

Target placement:

fz1=3Hz
fz2=12Hz
fp1=50Hz
fp2=17Hz

Important SLVA662 mapping:

R2,C1,C3→fz1,fp1
R3,C2,R1,R4→fz2,fp2

4. Intermediate SLVA662 factors

a=1+fcfp22
a=1+17172
a=1.4142
b=\sqrt{1+\left(\frac{f_c}{f_{p1}\right)^2}
b=1+17502
b=1.0562
c=1+fz1fc2
c=1+3172
c=1.0155
d=\sqrt{1+\left(\frac{f_c}{f_{z2}\right)^2}
d=1+17122
d=1.7341
aa=a⋅bc⋅d
aa=1.4142⋅1.05621.0155⋅1.7341
aa=0.8483

5. Calculate R2

SLVA662:

bb=Gfp2(R1+R4)R4gm(fp2-fz1)
bb=0.093187⋅17(3.63M+23.2k)23.2k⋅100μS⋅(17-3)
bb=156.10kΩ
R2=aa⋅bb
R2=0.8483⋅156.10k
R2=132.42kΩ

6. Calculate R3

SLVA662:

R3=R12fz2-R1R4(fp1-fz2)fp1-fz2)(R1+R4
R3=3.63M)2(12)-3.63M(23.2k)(50-1250-12)(3.63M+23.2k
R3=8.6336MΩ

7. Calculate C1

C1=12πfz1R2
C1=12π⋅3⋅132.42k
C1=400.62nF

8. Calculate C2

C2=12πfz2(R3+R1)
C2=12π⋅12(8.6336M+3.63M)
C2=1.081nF

9. Calculate C3

SLVA662:

C3=C12πC1R2fp1-1
C3=400.62nF2π⋅400.62nF⋅132.42k⋅50-1
C3=25.57nF

10. Final calculated physical OTA Type-III values

| Component | Calculated |
| --- | --- |
| R2 | 132.42 kΩ |
| R3 | 8.6336 MΩ |
| C1 | 400.62 nF |
| C2 | 1.081 nF |
| C3 | 25.57 nF |

Recommended standard values:

| Component | Standard value |
| --- | --- |
| R2 | 133 kΩ |
| R3 | 8.66 MΩ |
| C1 | 390 nF or 400 nF |
| C2 | 1.1 nF |
| C3 | 27 nF |

11. Pole-zero verification from calculated values

Using calculated values:

fz1=12πR2C1=3.000Hz
fz2=12π(R1+R3)C2=12.000Hz
fp1=C1+C32πR2C1C3=50.000Hz
fp2=12πR4R1R4+R1R3C2=17.000Hz

Final answer

Using Method B, where the feedback divider is removed from Tv(s)and included in the SLVA662 OTA Type-III compensator:

R2=132.42kΩ,R3=8.6336MΩ
C1=400.62nF,C2=1.081nF,C3=25.57nF

Recommended practical set:

R2=133kΩ,R3=8.66MΩ,C1=390nF,C2=1.1nF,C3=27nF

Corrected Method B result: the feedback divider is inside the SLVA662 OTA Type-III compensator, not separately inside Tv(s). SLVA662 page 11–12 shows that OTA Type-III includes R1, R4, gm, R2, R3, C1, C2, and C3together.

Use:

R1=RFB,TOP=3.63MΩ
R4=RFB,BOTTOM=23.2kΩ
GMV=100μS
fz1=3Hz,fz2=12Hz,fp1=50Hz,fp2=17Hz

Corrected summary table

| Vin | Pout | ∣Tv,base∣at 17 Hz | Required G=1/∣Tv,base∣| R2| R3| C1| C2| C3|
|---:|---:|---:|---:|---:|---:|---:|---:|---:|
| 90 Vac | 1700 W | 5.164 | 0.19365 | 275.18 kΩ | 8.6336 MΩ | 192.79 nF | 1.081 nF | 12.31 nF |
| 110 Vac | 1700 W | 5.161 | 0.19376 | 275.34 kΩ | 8.6336 MΩ | 192.68 nF | 1.081 nF | 12.30 nF |
| 120 Vac | 1700 W | 5.159 | 0.19384 | 275.45 kΩ | 8.6336 MΩ | 192.60 nF | 1.081 nF | 12.29 nF |
| 132 Vac | 1700 W | 5.157 | 0.19391 | 275.55 kΩ | 8.6336 MΩ | 192.53 nF | 1.081 nF | 12.29 nF |
| 180 Vac | 3600 W | 10.731 | 0.09319 | 132.42 kΩ | 8.6336 MΩ | 400.62 nF | 1.081 nF | 25.57 nF |
| 210 Vac | 3600 W | 10.712 | 0.09335 | 132.66 kΩ | 8.6336 MΩ | 399.91 nF | 1.081 nF | 25.53 nF |
| 220 Vac | 3600 W | 10.706 | 0.09341 | 132.73 kΩ | 8.6336 MΩ | 399.69 nF | 1.081 nF | 25.51 nF |
| 264 Vac | 3600 W | 10.672 | 0.09370 | 133.16 kΩ | 8.6336 MΩ | 398.42 nF | 1.081 nF | 25.43 nF |

Recommended hardware values for 3600 W design point

Use the high-power region because it sets the 17 Hz crossover at full power:

R2=133kΩ
R3=8.66MΩ
C1=390nF or 400nF
C2=1.1nF
C3=27nF

This matches the earlier corrected value set:

R2=133kΩ,R3=8.66MΩ,C1=390nF,C2=1.1nF,C3=27nF

if we use final values suggested (above), what will be the crossover frequency at low line and high line?

Using final values:

R2=133kΩ,R3=8.66MΩ,C1=390nF,C2=1.1nF,C3=27nF

the voltage-loop crossover becomes:

| Input | Pout | Gain at 17 Hz | Actual crossover |
| --- | --- | --- | --- |
| 90 Vac | 1700 W | -6.37 dB | 7.86 Hz |
| 110 Vac | 1700 W | -6.38 dB | 7.85 Hz |
| 120 Vac | 1700 W | -6.38 dB | 7.85 Hz |
| 132 Vac | 1700 W | -6.39 dB | 7.85 Hz |
| 180 Vac | 3600 W | -0.02 dB | 16.94 Hz |
| 210 Vac | 3600 W | -0.04 dB | 16.91 Hz |
| 220 Vac | 3600 W | -0.04 dB | 16.90 Hz |
| 264 Vac | 3600 W | -0.07 dB | 16.84 Hz |

So:

Low-line / 1700 W crossover ≈7.85Hz
High-line / 3600 W crossover ≈16.9Hz

This is expected because the compensator was sized for the 3600 W high-power design point, so at 1700 W the loop gain is lower and crossover shifts down.

There are two equations for Gmod in above image. Use top equation for loops calculation. For lowline get Gmod from TOP equation and place that value to bottom equation to get Rlpk value.