PFC Boost Inductor — Corrected Magnetic Design & Loss Model

Magnetics EDGE® 0059214A2 × 3-Stack (75 µ) · 2-Phase Interleaved CCM PFC · Per-Inductor Analysis

V_out = 394 Vdc · f_sw = 70 kHz · f_line = 60 Hz · N = 31 turns · validated EDGE 75 µ material model · iGSE cycle-averaged losses

Supersedes the magnetic sections (Ch. 3–4) of report pfc-1780893861522. Methodology follows the validated EDGE reference workflow; all material constants are taken from the Magnetics EDGE powder-core bulletin (2023) for the 75 µ toroid grade. No approximations or placeholder constants are used.

1) Corrections Applied vs. the Original Report

Each row fixes a specific defect found in the original Chapter 3–4 calculation. Items 1–3 change the numerical results; items 4–8 fix method/data integrity.

| # | Defect in original | Correction |
| --- | --- | --- |
| 1 | L-target derivation had a ×2 error: ripple ratio applied to per-phase I_φ,pk and mislabelled "478.64 → 239 µH (−50 %)". | Single definition ΔI_in,pp = r·I_in,pk → ΔI_L,pp = ΔI_in,pp/K(D) → L_φ. Gives 239 µH directly. |
| 2 | Per-phase RMS current overstated as 12.29 A (contradicted its own Table 3.2.2b = 10.07 A). | Correct I_φ,rms = 10.06 A at 90 V → copper loss ÷1.49. Matches LF+HF integration. |
| 3 | DC-bias current I_φ,avg@crest = 7.08 A — a second ×2 error (used I_pk,line/4). | Per-phase crest current = I_pk,line/2 = 14.15 A → H = 38.6 Oe (was 19.3 Oe). |
| 4 | B_sat = 1.05 T placeholder for EDGE. | Datasheet B_sat = 1.5 T (EDGE FeNi alloy). |
| 5 | Core-loss / DC-bias from an unstated "material database". | Catalog-anchored EDGE 75 µ constants (Step 3), reproducing published curves. |
| 6 | Worst case framed as 90 V (max D). | Flux, core loss and B_max worst case is D = 0.5 / 180 V; sizing-for-input-ripple worst case is 90 V — both stated. |
| 7 | Copper loss used I_total²·R_dc only; HF component ignored. | Explicit P_cu = I_φ,rms²·R_dc + I_hf²·(R_ac−R_dc), R_ac/R_dc≈1.15. |
| 8 | Biased L verified only at no-load (L0,min = 247.9 µH). | L_full(H) verified at peak bias, all 9 points: min 242.8 µH ≥ 239 µH. |

2) EDGE 75 µ Material Model (Catalog-Anchored)

Source: Magnetics EDGE Powder Cores Bulletin (2023), 75 µ toroid grade. Alloy FeNi, B_sat = 1.5 T, T_curie = 500 °C.

2.1  Core-loss density (sinusoidal-fit catalog curve):

P_L [mW/cm³] = a · B_ac,pk[T]^b · f[kHz]^c

a = 121.47,  b = 2.263,  c = 1.403

Validation: at 0.1 T, 100 kHz → 121.47·0.1^2.263·100^1.403 = 424 mW/cm³ (typical curve). A max-loss column uses ×1.286 (catalog typ→max ratio).

2.2  Permeability vs DC bias (toroid fit):

%µ_i = 100 / (0.01 + 1.58×10⁻⁹ · H^3.067)       H in Oe

H[Oe] = 0.4π · N · I / L_e[cm

L_full(H) = k(H) · L0,   k = %µi/100

2.3  iGSE triangular-ripple correction (replaces sinusoidal assumption):

F(D) = 2^c·[D^(1−c)+(1−D)^(1−c)] / [(2π)^(c−1)·∫₀^2π|cosθ|^c dθ],

∫ = 3.5794 (c=1.403)

| D | 0.354 (180 V crest) | 0.500 (ripple max) | 0.677 (90 V crest) |
| --- | --- | --- | --- |
| F(D) | 0.958 | 0.932 | 0.968 |

2.4  Powder-core note: DC bias produces NO additional core loss (Magnetics powder cores); bias acts only through permeability roll-off. No SPG/premagnetization loss adder is applied.

3) Reference Operating Point and Per-Phase Currents

Per-phase current at the line crest = I_pk,line / 2 (the two interleaved phases split the input current). I_φ,rms includes the switching-ripple (HF) component.

P_in = P_out/η   V_pk = √2·V_in   I_in,rms = P_in/(V_in·PF)

I_in,pk = √2·I_in,rms

I_φ,crest = I_in,pk/2     D_crest = 1 − V_pk/V_out

| V_in | V_pk(V) | I_in,rms | I_in,pk | I_φ,crest | D_crest | I_φ,rms | I_hf,rms |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 90 | 127.3 | 20.01 | 28.30 | 14.152 | 0.6770 | 10.058 | 1.008 |
| 110 | 155.6 | 16.21 | 22.92 | 11.459 | 0.6052 | 8.181 | 1.125 |
| 120 | 169.7 | 14.70 | 20.79 | 10.396 | 0.5693 | 7.444 | 1.172 |
| 132 | 186.7 | 13.24 | 18.72 | 9.359 | 0.5262 | 6.729 | 1.216 |
| 180 | 254.6 | 20.96 | 29.64 | 14.820 | 0.3539 | 10.558 | 1.291 |
| 200 | 282.8 | 18.68 | 26.41 | 13.207 | 0.2821 | 9.424 | 1.258 |
| 220 | 311.1 | 16.97 | 24.00 | 11.999 | 0.2103 | 8.569 | 1.202 |
| 230 | 325.3 | 16.18 | 22.89 | 11.444 | 0.1744 | 8.175 | 1.166 |
| 264 | 373.4 | 14.47 | 20.46 | 10.231 | 0.0524 | 7.307 | 1.027 |

Note: the 180–264 V rows are a separate power corner (3600 W). Currents are highest at 180 V — that corner, not 90 V, drives the worst loss/thermal case.

4) Inductance & DC-Bias Retention (All 9 Points)

No-load band: L0 = 247.9 / 271.0 / 294.1 µH (AL −8%/nom/+8%, N = 31). Biased inductance L_full = k(H)·L0. Target 239 µH per phase.

| V_in | I_φ,crest(A) | H(Oe) | k(H)% | L_min(µH) | L_nom(µH) | L_max(µH) |
| --- | --- | --- | --- | --- | --- | --- |
| 90 | 14.15 | 38.6 | 98.86 | 245.1 | 267.9 | 290.7 |
| 110 | 11.46 | 31.2 | 99.40 | 246.4 | 269.4 | 292.3 |
| 120 | 10.40 | 28.3 | 99.55 | 246.8 | 269.8 | 292.8 |
| 132 | 9.36 | 25.5 | 99.68 | 247.1 | 270.1 | 293.1 |
| 180 | 14.82 | 40.4 | 98.69 | 244.6 | 267.4 | 290.2 |
| 200 | 13.21 | 36.0 | 99.07 | 245.6 | 268.5 | 291.4 |
| 220 | 12.00 | 32.7 | 99.31 | 246.2 | 269.1 | 292.1 |
| 230 | 11.44 | 31.2 | 99.40 | 246.4 | 269.4 | 292.3 |
| 264 | 10.23 | 27.9 | 99.57 | 246.8 | 269.8 | 292.8 |

Worst-case minimum-L guarantee (AL −8%, peak instantaneous bias incl. ½-ripple): L_full,min = 242.8 µH at 180 V ≥ 239 µH target → PASS, but margin is only ~1.6 % (the no-load check alone hid this). Bias retention ≥ 97.9 % everywhere; the 75 µ knee (80 % at ~95 Oe) is far from the ~47 Oe operating peak.

5) Flux Density (All 9 Points)

ΔB_pp(θ) = V_in(θ)·D(θ) / (N·A_e·f_sw)

B_ac,pk = ΔB_pp/2   (volt-second set; independent of L)

B_dc = L·i_φ,avg/(N·A_e)

B_max = L·(i_φ,avg + ½ΔI_pp)/(N·A_e)

| V_in | D_crest | B_ac,pk@crest(T) | B_ac,pk,max(T) | B_dc,pk(T) | B_max(T) |
| --- | --- | --- | --- | --- | --- |
| 90 | 0.677 | 0.0460 | 0.0460 | 0.283 | 0.329 |
| 110 | 0.605 | 0.0502 | 0.0502 | 0.230 | 0.281 |
| 120 | 0.569 | 0.0515 | 0.0515 | 0.209 | 0.261 |
| 132 | 0.526 | 0.0524 | 0.0524 | 0.189 | 0.241 |
| 180 | 0.354 | 0.0481 | 0.0525 | 0.296 | 0.344 |
| 200 | 0.282 | 0.0426 | 0.0525 | 0.265 | 0.307 |
| 220 | 0.210 | 0.0349 | 0.0525 | 0.241 | 0.276 |
| 230 | 0.174 | 0.0303 | 0.0525 | 0.230 | 0.260 |
| 264 | 0.052 | 0.0104 | 0.0525 | 0.206 | 0.217 |

B_ac,pk maximises at V_in = V_out/2 = 197 V (D = 0.5), reached twice per half-cycle at high line (double-hump). Worst B_max = 0.344 T at 180 V vs B_sat = 1.5 T → 336 % saturation margin (the original used B_max = 0.3305 T evaluated at the wrong 90 V point and B_sat = 1.05 T).

6) Copper Loss (Litz, HF Term Explicit)

MLT(toroid) = 2·(w + S·HT) + build = 2·(11.65 + 44.58) + 3.8 = 116.3 mm/turn   (NOT π·D_mean)

ℓ_cu = N·MLT = 3.604 m

A_cu = 2×800×(π/4)(0.1)² = 12.57 mm²

DCR_25/100 = 5.03 / 6.51 mΩ

P_cu(T) = I_φ,rms²·DCR(T) + I_hf,rms²·(R_ac−R_dc),

R_ac/R_dc ≈ 1.15   (δ_70k = 0.286 mm ≫ 0.1 mm strand)

| V_in | I_φ,rms(A) | I_hf,rms(A) | P_cu@25°C(W) | P_cu@100°C(W) |
| --- | --- | --- | --- | --- |
| 90 | 10.058 | 1.008 | 0.509 | 0.660 |
| 110 | 8.181 | 1.125 | 0.337 | 0.437 |
| 120 | 7.444 | 1.172 | 0.280 | 0.362 |
| 132 | 6.729 | 1.216 | 0.229 | 0.296 |
| 180 | 10.558 | 1.291 | 0.562 | 0.727 |
| 200 | 9.424 | 1.258 | 0.448 | 0.580 |
| 220 | 8.569 | 1.202 | 0.370 | 0.480 |
| 230 | 8.175 | 1.166 | 0.337 | 0.436 |
| 264 | 7.307 | 1.027 | 0.269 | 0.349 |

Current density J = 10.56 A/mm² ÷ ... = 0.84 A/mm² at worst RMS (both parallel conductors counted) — far below the 3–7 A/mm² target; the winding is heavily over-coppered (optimisation headroom). The original report’s J = 1.59 A/mm² and the I_rms = 0.000 A in its sizing table were artifacts of the RMS error in correction #2.

7) Core Loss — Cycle-Averaged iGSE (All 9 Points)

B_ac,pk(t) = V_in(t)·D(t)/(2·N·A_e·f_sw)

P_core(t) = a·B_ac,pk(t)^b·(f_sw/1000)^c·F(D(t))·V_e/1000  [W]

P_core,avg = (1/T_half)·∫ P_core(t) dt        (V_e = 62.1 cm³)

| V_in | B_ac,pk@crest(T) | P_core,avg typ(W) | P_core,avg max(W) | P_core,pk(W) |
| --- | --- | --- | --- | --- |
| 90 | 0.0460 | 1.545 | 1.986 | 2.660 |
| 110 | 0.0502 | 1.940 | 2.494 | 3.169 |
| 120 | 0.0515 | 2.106 | 2.709 | 3.336 |
| 132 | 0.0524 | 2.273 | 2.923 | 3.447 |
| 180 | 0.0481 | 2.509 | 3.226 | 3.466 |
| 200 | 0.0426 | 2.405 | 3.093 | 3.466 |
| 220 | 0.0349 | 2.206 | 2.838 | 3.466 |
| 230 | 0.0303 | 2.082 | 2.677 | 3.466 |
| 264 | 0.0104 | 1.622 | 2.086 | 3.466 |

Cross-check: the cycle-averaged 90 V figure (1.55 W typ) reproduces the original report’s own Chapter-4 iGSE value (1.546 W) — confirming the engine. The peak-point method the original used in Chapter 3 (2.64 W at 90 V) overstates low-line loss by ~70 % because D is far from 0.5; the cycle average is the correct number.

8) Total Loss and Temperature Rise (All 9 Points)

ΔT[°C] = (P_total[mW] / SA[cm²])^0.833

SA = 204 cm² (assembled OD 63.6 × H 54.2 mm, natural convection)

| V_in | P_core,max(W) | P_cu,100°C(W) | P_total,typ(W) | P_total,max(W) | ΔT_max(°C) |
| --- | --- | --- | --- | --- | --- |
| 90 | 1.99 | 0.66 | 2.20 | 2.65 | 8.5 |
| 110 | 2.49 | 0.44 | 2.38 | 2.93 | 9.2 |
| 120 | 2.71 | 0.36 | 2.47 | 3.07 | 9.6 |
| 132 | 2.92 | 0.30 | 2.57 | 3.22 | 10.0 |
| 180 | 3.23 | 0.73 | 3.24 | 3.95 | 11.8 |
| 200 | 3.09 | 0.58 | 2.98 | 3.67 | 11.1 |
| 220 | 2.84 | 0.48 | 2.69 | 3.32 | 10.2 |
| 230 | 2.68 | 0.44 | 2.52 | 3.11 | 9.7 |
| 264 | 2.09 | 0.35 | 1.97 | 2.44 | 7.9 |

Worst case 180 V: P_total = 3.24 W (typ) / 3.95 W (max) per inductor, ΔT ≈ 11.8 °C (natural convection, T_a = 50 °C → winding ≈ 62 °C). Core and copper are co-dominant. Fan cooling (per spec) gives substantial additional margin against the 110 °C hotspot limit / 200 °C coating rating.

9) Saturation, Margins & Verdict

| Check | Value | Limit | Verdict |
| --- | --- | --- | --- |
| Min biased inductance (AL−8%, peak bias) | 242.8 µH | ≥ 239 µH | PASS (~1.6% margin) |
| Peak flux density B_max (180 V) | 0.344 T | B_sat = 1.5 T | PASS (336% margin) |
| DC-bias retention (worst, 180 V peak) | 97.9 % | — (knee ~95 Oe) | PASS |
| Window fill (bare Cu / insulated) | ~41 % / ~57 % | ≤ 45 % / ≤ 65 % | PASS |
| Worst-case ΔT (180 V, max loss, nat. conv.) | 11.8 °C | 60 °C budget | PASS (80% margin) |
| Current density (worst RMS) | 0.84 A/mm² | 3–7 A/mm² target | PASS (over-coppered) |

Net: the design passes on every magnetic criterion. The corrections do not flip any verdict, but they change the numbers materially — copper loss drops ~33–48 %, core loss at low line drops ~40 % vs the Chapter-3 peak-point figures, the true worst-case corner moves to 180 V, and the inductance margin is revealed as ~1.6 % (not the comfortable figure the no-load check implied). The biggest residual uncertainty is the core-loss anchor (typ vs max, ±29 %); confirm the 75 µ datasheet maximum before release.